Document Type: Detail Design

Document Name:

ADSP FRAMEWORK: ADMA DRIVER

Renesas Electronics Corporation

Renesas Design Vietnam Co. Ltd

R-Car Software Solution 2 Group

Middleware 1 Team

Table of Contents

1.	Overview	3

2.	Function list	4

3.	Detail information	5

3.1	Data type definition	5

3.1.1	ADMA_CH	5

3.1.2	dmac_error_code	6

3.1.3	ADMA_STAGE	6

3.1.4	DMAC_SRCDST	7

3.1.5	ADMAC_MODE	10

3.1.6	DMAC_SETTING	10

3.1.7	DMAC_AUDIO_MODULE_SETTING	11

3.2	Macro definition	12

3.3	Register definition	14

3.4	Global variable definition	15

3.5	Function definition	18

3.5.1	admac_get_register_addr	18

3.5.2	admac_set_stage_number	20

3.5.3	admac_reset_descriptor	21

3.5.4	admac_start_descriptor	22

3.5.5	admac_enable_ch	23

3.5.6	admac_disable_ch	24

3.5.7	admac_enable_module	25

3.5.8	admac_clear_channel	26

3.5.9	admac_check_stage	27

3.5.10	admac_check_and_clear_interrupt	28

3.5.11	admac_start	29

3.5.12	admac_stop	30

3.5.13	admac_descriptor_setup	31

3.5.14	admac_stage_config	32

3.5.15	admac_pre_config	33

3.5.16	admac_post_config	35

3.5.17	admac_check_valid_channel	37

3.5.18	admac_setting_use_descriptor	39

3.5.19	admac_setting_nouse_descriptor	40

3.5.20	admac_resetup	42

3.5.21	admac_pause	43

4.	Revision history	44

List of Figures

Figure 1-1 The software architecture	3

Figure 3-1 admac_get_register_addr flowchart	19

Figure 3-2 admac_set_stage_number flowchart	20

Figure 3-3 admac_reset_descriptor flowchart	21

Figure 3-4 admac_start_desciptor flowchart	22

Figure 3-5 admac_enable_ch flowchart	23

Figure 3-6 admac_disable_ch flowchart	24

Figure 3-7 admac_enable_module flowchart	25

Figure 3-8 admac_clear_channel flowchart	26

Figure 3-9 admac_check_stage flowchart	27

Figure 3-10 admac_check_and_clear_interrupt flowchart	28

Figure 3-11admac_start flowchart	29

Figure 3-12 admac_stop flowchart	30

Figure 3-13 admac_descriptor_setup flowchart	31

Figure 3-14 admac_stage_config flowchart	32

Figure 3-15 admac_pre_config flowchart	34

Figure 3-16 admac_post_config flowchart	35

Figure 3-17 admac_check_valid_channel flowchart	38

Figure 3-18 admac_setting_use_descriptor flowchart	39

Figure 3-19 admac_setting_nouse_descriptor flowchart	41

Figure 3-20 admac_resetup flowchart	42

Figure 3-21 admac_pause flowchart	43

List of Table

Table 2-1 Function list	4

Table 3-1 ADMA_CH type information	5

Table 3-2 dmac_error_code type information	6

Table 3-3 ADMA_STAGE type information	6

Table 3-4 DMAC_SRCDST type information	7

Table 3-5 ADMAC_MODE type information	10

Table 3-6 DMAC_SETTING type information	10

Table 3-7 DMAC_AUDIO_MODULE_SETTING type information	11

Table 2-1 Function list	4

Table 3-1 ADMA_CH type information	5

Table 3-2 dmac_error_code type information	6

Table 3-3 ADMA_STAGE type information	6

Table 3-4 DMAC_SRCDST type information	7

Table 3-5 ADMAC_MODE type information	10

Table 3-6 DMAC_SETTING type information	10

Table 3-7 DMAC_AUDIO_MODULE_SETTING type information	11

# Overview

This section provides an overview of software architecture.

User Application

ADSP Interface

Kernel Space

User Space

ADSP Driver

ARM

Audio HW

ADSP Framework

TDM class

Renderer class

Equalizer class

Capture class

TDM Plugin

ADSP

Plugin

Equalizer Plugin*

Capture Plugin

Renderer Plugin

This document’s target is in side of red square

* not connect to hardware modules

- DAC/

- ADC

FIFO

SCU

PDMA

SSI

SSIU

ADMA

Figure 1-1 The software architecture

# Function list

The following is list of functions:

Table 2-1 Function list

| Type | Function Name | Outline |
| --- | --- | --- |
| API | admac_get_register_addr | This API is to get DMAC channel N register’s address |
|  | admac_check_stage | This API is to check the current stage of DMAC module |
|  | admac_check_and_clear_interrupt | This API is to check and clear interrupt of DMAC channel N |
|  | admac_start | This API is to start DMAC channel N |
|  | admac_stop | This API is to stop DMAC channel N |
|  | admac_check_valid_channel | This API is to check availability of DMAC module and choose an available one |
|  | admac_setting_use_descriptor | This API is to set DMAC module using descriptor memory |
|  | admac_setting_nouse_descriptor | This API is to set DMAC module not using descriptor memory |
|  | admac_resetup | This API is to re-setup DMAC module |
|  | admac_pause | This API is to pause ADMAC channel N |
| Internal function | admac_set_stage_number | This function is to set stage number |
|  | admac_reset_descriptor | This function is to reset descriptor |
|  | admac_start_descriptor | This function is to start descriptor |
|  | admac_enable_ch | This function is to enable a channel |
|  | admac_disable_ch | This function is to disable a channel |
|  | admac_enable_module | This function is to enable DMAC module |
|  | admac_clear_channel | This function is to clear value set in DMAC channel N register |
|  | admac_descriptor_setup | This function is to set up DMAC descriptor |
|  | admac_stage_config | This function is to set DMAC stage |
|  | admac_pre_config | This function is to set DMAC parameters |
|  | admac_post_config | This function is to post-set DMAC parameters |

# Detail information

This section describes detail information of data types, macro definitions, implemented APIs and internal function units, global variables.

## Data type definition

### ADMA_CH

The data type ADMA_CH is a type-defined enumeration that lists all supported ADMAC channels.

Table 3-1 ADMA_CH type information

| Member name | Value | Outline |
| --- | --- | --- |
| ADMAC_CH00 | 0 | ADMAC channel 0 |
| ADMAC_CH01 | 1 | ADMAC channel 1 |
| ADMAC_CH02 | 2 | ADMAC channel 2 |
| ADMAC_CH03 | 3 | ADMAC channel 3 |
| ADMAC_CH04 | 4 | ADMAC channel 4 |
| ADMAC_CH05 | 5 | ADMAC channel 5 |
| ADMAC_CH06 | 6 | ADMAC channel 6 |
| ADMAC_CH07 | 7 | ADMAC channel 7 |
| ADMAC_CH08 | 8 | ADMAC channel 8 |
| ADMAC_CH09 | 9 | ADMAC channel 9 |
| ADMAC_CH10 | 10 | ADMAC channel 10 |
| ADMAC_CH11 | 11 | ADMAC channel 11 |
| ADMAC_CH12 | 12 | ADMAC channel 12 |
| ADMAC_CH13 | 13 | ADMAC channel 13 |
| ADMAC_CH14 | 14 | ADMAC channel 14 |
| ADMAC_CH15 | 15 | ADMAC channel 15 |
| ADMAC_CH16 | 16 | ADMAC channel 16 |
| ADMAC_CH17 | 17 | ADMAC channel 17 |
| ADMAC_CH18 | 18 | ADMAC channel 18 |
| ADMAC_CH19 | 19 | ADMAC channel 19 |
| ADMAC_CH20 | 20 | ADMAC channel 20 |
| ADMAC_CH21 | 21 | ADMAC channel 21 |
| ADMAC_CH22 | 22 | ADMAC channel 22 |
| ADMAC_CH23 | 23 | ADMAC channel 23 |
| ADMAC_CH24 | 24 | ADMAC channel 24 |
| ADMAC_CH25 | 25 | ADMAC channel 25 |
| ADMAC_CH26 | 26 | ADMAC channel 26 |
| ADMAC_CH27 | 27 | ADMAC channel 27 |
| ADMAC_CH28 | 28 | ADMAC channel 28 |
| ADMAC_CH29 | 29 | ADMAC channel 29 |
| ADMAC_CH30 | 30 | ADMAC channel 30 |
| ADMAC_CH31 | 31 | ADMAC channel 31 |
| ADMAC_CHMAX | 32 | Total number of ADMAC channels |

### dmac_error_code

The data type dmac_error_code is a type-defined enumeration that lists all error codes for ADMAC.

Table 3-2 dmac_error_code type information

| Member name | Value |
| --- | --- |
| DMAC_ERROR_NONE | 0 |
| DMAC_ERROR_BUSY | -1 |
| DMAC_ERROR_INVALID | -2 |
| DMAC_ERROR_OUT_RANGE | -3 |

### ADMA_STAGE

The data type ADMA_STAGE is a type-defined enumeration of stages of ADMAC.

Table 3-3 ADMA_STAGE type information

| Member name | Value | Outline |
| --- | --- | --- |
| ADMAC_STAGE0 | 0 | ADMAC stage 0 |
| ADMAC_STAGE1 | 1 | ADMAC stage 1 |
| ADMAC_STAGEMAX | 2 | Total stages of ADMAC |

### DMAC_SRCDST

The data type DMAC_SRCDST is a type-defined enumeration that shows connections between ADMAC and other devices.

Table 3-4 DMAC_SRCDST type information

| Member name | Value |
| --- | --- |
| DMAC_SSI00 | DMAC_SSI_MODULE_MIN |
| DMAC_SSI01 | 1 |
| DMAC_SSI02 | 2 |
| DMAC_SSI03 | 3 |
| DMAC_SSI04 | 4 |
| DMAC_SSI05 | 5 |
| DMAC_SSI06 | 6 |
| DMAC_SSI07 | 7 |
| DMAC_SSI10 | 10 |
| DMAC_SSI11 | 11 |
| DMAC_SSI12 | 12 |
| DMAC_SSI13 | 13 |
| DMAC_SSI14 | 14 |
| DMAC_SSI15 | 15 |
| DMAC_SSI16 | 16 |
| DMAC_SSI17 | 17 |
| DMAC_SSI20 | 20 |
| DMAC_SSI21 | 21 |
| DMAC_SSI22 | 22 |
| DMAC_SSI23 | 23 |
| DMAC_SSI24 | 24 |
| DMAC_SSI25 | 25 |
| DMAC_SSI26 | 26 |
| DMAC_SSI27 | 27 |
| DMAC_SSI30 | 30 |
| DMAC_SSI31 | 31 |
| DMAC_SSI32 | 32 |
| DMAC_SSI33 | 33 |
| DMAC_SSI34 | 34 |
| DMAC_SSI35 | 35 |
| DMAC_SSI36 | 36 |
| DMAC_SSI37 | 37 |
| DMAC_SSI40 | 40 |
| DMAC_SSI41 | 41 |
| DMAC_SSI42 | 42 |
| DMAC_SSI43 | 43 |
| DMAC_SSI44 | 44 |
| DMAC_SSI45 | 45 |
| DMAC_SSI46 | 46 |
| DMAC_SSI47 | 47 |
| DMAC_SSI5 | 50 |
| DMAC_SSI6 | 60 |
| DMAC_SSI7 | 70 |
| DMAC_SSI8 | 80 |
| DMAC_SSI90 | 90 |
| DMAC_SSI91 | 91 |
| DMAC_SSI92 | 92 |
| DMAC_SSI93 | 93 |
| DMAC_SSI94 | 94 |
| DMAC_SSI95 | 95 |
| DMAC_SSI96 | 96 |
| DMAC_SSI97 | DMAC_SSI_MODULE_MAX |
| DMAC_DTCPPP0 | 98 |
| DMAC_DTCPPP1 | 99 |
| DMAC_DTCPCP0 | 100 |
| DMAC_DTCPCP1 | 101 |
| DMAC_SCU_SRCI0 | DMAC_SCU_SRC_INPUT_MODULE_MIN |
| DMAC_SCU_SRCI1 | 111 |
| DMAC_SCU_SRCI2 | 112 |
| DMAC_SCU_SRCI3 | 113 |
| DMAC_SCU_SRCI4 | 114 |
| DMAC_SCU_SRCI5 | 115 |
| DMAC_SCU_SRCI6 | 116 |
| DMAC_SCU_SRCI7 | 117 |
| DMAC_SCU_SRCI8 | 118 |
| DMAC_SCU_SRCI9 | DMAC_SCU_SRC_INPUT_MODULE_MAX |
| DMAC_SCU_SRCO0 | DMAC_SCU_SRC_OUTPUT_MODULE_MIN |
| DMAC_SCU_SRCO1 | 121 |
| DMAC_SCU_SRCO2 | 122 |
| DMAC_SCU_SRCO3 | 123 |
| DMAC_SCU_SRCO4 | 124 |
| DMAC_SCU_SRCO5 | 125 |
| DMAC_SCU_SRCO6 | 126 |
| DMAC_SCU_SRCO7 | 127 |
| DMAC_SCU_SRCO8 | 128 |
| DMAC_SCU_SRCO9 | DMAC_SCU_SRC_OUTPUT_MODULE_MAX |
| DMAC_SCU_CMD0 | DMAC_SCU_CMD_MODULE_MIN |
| DMAC_SCU_CMD1 | DMAC_SCU_CMD_MODULE_MAX |
| DMAC_MLM0 | 132 |
| DMAC_MLM1 | 133 |
| DMAC_MLM2 | 134 |
| DMAC_MLM3 | 135 |
| DMAC_MLM4 | 136 |
| DMAC_MLM5 | 137 |
| DMAC_MLM6 | 138 |
| DMAC_MLM7 | 139 |
| DMAC_NONCONTROL | 140 |
| DMAC_MAX | 141 |

### ADMAC_MODE

The data type ADMAC_MODE is a type-defined enumeration of ADMAC operation mode.

Table 3-5 ADMAC_MODE type information

| Member name | Value | Outline |
| --- | --- | --- |
| ADMAC_NORMAL_MODE | 1 | ADMAC normal mode |
| ADMAC_REPEAT_MODE | 2 | ADMAC repeating mode |

### DMAC_SETTING

The data type DMAC_SETTING is a type-defined structure that possesses necessary parameters for DMAC module.

Table 3-6 DMAC_SETTING type information

| Member name | Data type | Outline |
| --- | --- | --- |
| dst | WORD32 | Destination address |
| src | WORD32 | Source address |
| transmem | pVOID | Transmission memory address |
| trans_num | WORD32 | Transfer number |
| stg_num | WORD32 | Stage number |
| stg_size | WORD32 | Stage memory size |
| buf_size | WORD32 | Ring buffer size |
| trans_mode | WORD32 | Indicator of transfer/receive mode |
| mode | ADMAC_MODE | Operation mode |
| dma_ch | UWORD32 | ADMAC channel |
| src_reqst | UWORD32 | Source request |
| trans_size | UWORD32 | Transfer size |

### DMAC_AUDIO_MODULE_SETTING

The data type DMAC_AUDIO_MODULE_SETTING is a type-defined structure of ADMAC device information.

Table 3-7 DMAC_AUDIO_MODULE_SETTING type information

| Member name | Data type | Outline |
| --- | --- | --- |
| module_address | UWORD32 | Destination address |
| trans | UWORD32 | Source address |
| recei | UWORD32 | Transmission memory address |

## Macro definition

| Macros | Value | Outline |
| --- | --- | --- |
| DMAC_SSI_MODULE_MIN | 0 | SSI module min index |
| DMAC_SSI_MODULE_MAX | 97 | SSI module max index |
| DMAC_SCU_SRC_INPUT_MODULE_MIN | 110 | SCU input module min index |
| DMAC_SCU_SRC_INPUT_MODULE_MAX | 119 | SCU input module max index |
| DMAC_SCU_SRC_OUTPUT_MODULE_MIN | 120 | SCU output module min index |
| DMAC_SCU_SRC_OUTPUT_MODULE_MAX | 129 | SCU output module max index |
| DMAC_SCU_CMD_MODULE_MIN | 130 | CMD module min index |
| DMAC_SCU_CMD_MODULE_MAX | 131 | CMD module max index |
| XF_RCAR_DMAOR_L | XF_RCAR_REG_DMAOR_L | DMA operation register (for lower-numbered channels) |
| XF_RCAR_DMAOR_U | XF_RCAR_REG_DMAOR_U | DMA channel clear register (for lower-numbered channels) |
| XF_DMACHCLR_L | XF_RCAR_REG_DMACHCLR_L | DMA channel clear register (for lower-numbered channels) |
| XF_DMACHCLR_U | XF_RCAR_REG_DMACHCLR_U | DMA channel clear register (for higher-numbered channels) |
| XF_RCAR_DMARS_0 | XF_RCAR_REG_DMARS_0 | Channel 0 DMA Extended Resource Selectors Register |
| XF_RCAR_DMADPBASE_0 | XF_RCAR_REG_DMADPBASE_0 | Channel 0 DMA Descriptor Base Address Register |
| XF_RCAR_DMACHCRB_0 | XF_RCAR_REG_DMACHCRB_0 | Channel 0 DMA channel control register B |
| XF_RCAR_DMACHCR_0 | XF_RCAR_REG_DMACHCR_0 | Channel 0 DMA Channel Control Register |
| XF_RCAR_DMATCR_0 | XF_RCAR_REG_DMATCR_0 | Channel 0 DMA Transfer Count Register |
| XF_RCAR_DMABUFCR_0 | XF_RCAR_REG_DMABUFCR_0 | Channel 0 DMA Buffer Control register |
| XF_RCAR_DMASAR_0 | XF_RCAR_REG_DMASAR_0 | Channel 0 Source Address register |
| XF_RCAR_DMADAR_0 | XF_RCAR_REG_DMADAR_0 | Channel 0 Destination Address register |
| DMAC_TRANSMIT_REQ | 1 | source request is source |
| DMAC_RECEIVE_REQ | 2 | source request is destination |

## Register definition

Below is the table listing all items that are macros representing register addresses located in repository s492d/include/sys/xt-shmem/board-rcar/xf-registers.h

| Register | Outline |
| --- | --- |
| XF_RCAR_REG_DMAOR_L | DMA operation register (for lower-numbered channels) |
| XF_RCAR_REG_DMAOR_U | DMA channel clear register (for lower-numbered channels) |
| XF_RCAR_REG_DMACHCLR_L | DMA channel clear register (for lower-numbered channels) |
| XF_RCAR_REG_DMACHCLR_U | DMA channel clear register (for higher-numbered channels) |
| XF_RCAR_REG_DMARS_0 | Channel 0 DMA Extended Resource Selectors Register |
| XF_RCAR_REG_DMADPBASE_0 | Channel 0 DMA Descriptor Base Address Register |
| XF_RCAR_REG_DMACHCRB_0 | Channel 0 DMA channel control register B |
| XF_RCAR_REG_DMACHCR_0 | Channel 0 DMA Channel Control Register |
| XF_RCAR_REG_DMATCR_0 | Channel 0 DMA Transfer Count Register |
| XF_RCAR_REG_DMABUFCR_0 | Channel 0 DMA Buffer Control register |
| XF_RCAR_REG_DMASAR_0 | Channel 0 Source Address register |
| XF_RCAR_REG_DMADAR_0 | Channel 0 Destination Address register |

## Global variable definition

Below is the array of struct DMAC_AUDIO_MODULE_SETTING-datatype admac_set_value holds 141 (DMAC_MAX) elements representing settings for ADMAC data transfer

| static DMAC_AUDIO_MODULE_SETTING admac_set_value[DMAC_MAX] |  |  |  |
| --- | --- | --- | --- |
| Array’s index | module_address | trans | recei |
| DMAC_SSI00 | 0xEC100000 | 0x15 | 0x16 |
| DMAC_SSI01 | 0xEC100400 | 0x35 | 0x36 |
| DMAC_SSI02 | 0xEC100800 | 0x37 | 0x38 |
| DMAC_SSI03 | 0xEC100C00 | 0x47 | 0x48 |
| DMAC_SSI04 | 0xEC10A000 | 0x3F | 0x40 |
| DMAC_SSI05 | 0xEC10A400 | 0x43 | 0x44 |
| DMAC_SSI06 | 0xEC10A800 | 0x4F | 0x50 |
| DMAC_SSI07 | 0xEC10AC00 | 0x53 | 0x54 |
| DMAC_SSI10 | 0xEC101000 | 0x49 | 0x4A |
| DMAC_SSI11 | 0xEC101400 | 0x4B | 0x4C |
| DMAC_SSI12 | 0xEC101800 | 0x57 | 0x58 |
| DMAC_SSI13 | 0xEC101C00 | 0x59 | 0x5A |
| DMAC_SSI14 | 0xEC10B000 | 0x5F | 0x60 |
| DMAC_SSI15 | 0xEC10B400 | 0xC3 | 0xC4 |
| DMAC_SSI16 | 0xEC10B800 | 0xC7 | 0xC8 |
| DMAC_SSI17 | 0xEC10BC00 | 0xCB | 0xCC |
| DMAC_SSI20 | 0xEC102000 | 0x63 | 0x64 |
| DMAC_SSI21 | 0xEC102400 | 0x67 | 0x68 |
| DMAC_SSI22 | 0xEC102800 | 0x6B | 0x6C |
| DMAC_SSI23 | 0xEC102C00 | 0x6D | 0x6E |
| DMAC_SSI24 | 0xEC10C000 | 0xCF | 0xCE |
| DMAC_SSI25 | 0xEC10C400 | 0xEB | 0xEC |
| DMAC_SSI26 | 0xEC10C800 | 0xED | 0xEE |
| DMAC_SSI27 | 0xEC10CC00 | 0xEF | 0xF0 |
| DMAC_SSI30 | 0xEC103000 | 0x6F | 0x70 |
| DMAC_SSI31 | 0xEC103400 | 0x21 | 0x22 |
| DMAC_SSI32 | 0xEC103800 | 0x23 | 0x24 |
| DMAC_SSI33 | 0xEC103C00 | 0x25 | 0x26 |
| DMAC_SSI34 | 0xEC10D000 | 0x27 | 0x28 |
| DMAC_SSI35 | 0xEC10D400 | 0x29 | 0x2A |
| DMAC_SSI36 | 0xEC10D800 | 0x2B | 0x2C |
| DMAC_SSI37 | 0xEC10DC00 | 0x2D | 0x2E |
| DMAC_SSI40 | 0xEC104000 | 0x71 | 0x72 |
| DMAC_SSI41 | 0xEC104400 | 0x17 | 0x18 |
| DMAC_SSI42 | 0xEC104800 | 0x19 | 0x1A |
| DMAC_SSI43 | 0xEC104C00 | 0x1B | 0x1C |
| DMAC_SSI44 | 0xEC10E000 | 0x1D | 0x1E |
| DMAC_SSI45 | 0xEC10E400 | 0x1F | 0x20 |
| DMAC_SSI46 | 0xEC10E800 | 0x31 | 0x32 |
| DMAC_SSI47 | 0xEC10EC00 | 0x33 | 0x34 |
| DMAC_SSI5 | 0xEC105000 | 0x73 | 0x74 |
| DMAC_SSI6 | 0xEC106000 | 0x75 | 0x76 |
| DMAC_SSI7 | 0xEC107000 | 0x79 | 0x7A |
| DMAC_SSI8 | 0xEC108000 | 0x7B | 0x7C |
| DMAC_SSI90 | 0xEC109000 | 0x7D | 0x7E |
| DMAC_SSI91 | 0xEC109400 | 0x7F | 0x80 |
| DMAC_SSI92 | 0xEC109800 | 0x81 | 0x82 |
| DMAC_SSI93 | 0xEC109C00 | 0x83 | 0x84 |
| DMAC_SSI94 | 0xEC10F000 | 0xA3 | 0xA4 |
| DMAC_SSI95 | 0xEC10F400 | 0xA5 | 0xA6 |
| DMAC_SSI96 | 0xEC10F800 | 0xA7 | 0xA8 |
| DMAC_SSI97 | 0xEC10FC00 | 0xA9 | 0xAA |
| DMAC_DTCPPP0 | 0xEC120000 | 0xBF | 0xC0 |
| DMAC_DTCPPP1 | 0xEC120400 | 0xD5 | 0xD6 |
| DMAC_DTCPCP0 | 0xEC124000 | 0xD7 | 0xD8 |
| DMAC_DTCPCP1 | 0xEC124400 | 0xD9 | 0xDA |
| DMAC_SCU_SRCI0 | 0xEC000000 | 0x85 | 0xff |
| DMAC_SCU_SRCI1 | 0xEC000400 | 0x87 | 0xff |
| DMAC_SCU_SRCI2 | 0xEC000800 | 0x89 | 0xff |
| DMAC_SCU_SRCI3 | 0xEC000C00 | 0x8B | 0xff |
| DMAC_SCU_SRCI4 | 0xEC001000 | 0x8D | 0xff |
| DMAC_SCU_SRCI5 | 0xEC001400 | 0x8F | 0xff |
| DMAC_SCU_SRCI6 | 0xEC001800 | 0x91 | 0xff |
| DMAC_SCU_SRCI7 | 0xEC001C00 | 0x93 | 0xff |
| DMAC_SCU_SRCI8 | 0xEC002000 | 0x95 | 0xff |
| DMAC_SCU_SRCI9 | 0xEC002400 | 0x97 | 0xff |
| DMAC_SCU_SRCO0 | 0xEC004000 | 0xff | 0x9A |
| DMAC_SCU_SRCO1 | 0xEC004400 | 0xff | 0x9C |
| DMAC_SCU_SRCO2 | 0xEC004800 | 0xff | 0x9E |
| DMAC_SCU_SRCO3 | 0xEC004C00 | 0xff | 0xA0 |
| DMAC_SCU_SRCO4 | 0xEC005000 | 0xff | 0xB0 |
| DMAC_SCU_SRCO5 | 0xEC005400 | 0xff | 0xB2 |
| DMAC_SCU_SRCO6 | 0xEC005800 | 0xff | 0xB4 |
| DMAC_SCU_SRCO7 | 0xEC005C00 | 0xff | 0xB6 |
| DMAC_SCU_SRCO8 | 0xEC006000 | 0xff | 0xB8 |
| DMAC_SCU_SRCO9 | 0xEC006400 | 0xff | 0xBA |
| DMAC_SCU_CMD0 | 0xEC008000 | 0xff | 0xBC |
| DMAC_SCU_CMD1 | 0xEC008400 | 0xff | 0xBE |
| DMAC_MLM0 | 0xEC020000 | 0xDB | 0xDC |
| DMAC_MLM1 | 0xEC020400 | 0xE3 | 0xE4 |
| DMAC_MLM2 | 0xEC020800 | 0xE5 | 0xE6 |
| DMAC_MLM3 | 0xEC020C00 | 0xE7 | 0xE8 |
| DMAC_MLM4 | 0xEC021000 | 0xF3 | 0xF4 |
| DMAC_MLM5 | 0xEC021400 | 0xF5 | 0xF6 |
| DMAC_MLM6 | 0xEC021800 | 0xF7 | 0xF8 |
| DMAC_MLM7 | 0xEC021C00 | 0xF9 | 0xFA |

## Function definition

### admac_get_register_addr

DD_PLG_RDR_07_001

| Syntax | inline UWORD32 admac_get_register_addr(UWORD32 reg_addr0, ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to get DMAC channel N register’s address. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | UWORD32 | reg_addr0 | I | Pointer to register of channel 0 |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | The value of register channel N |  |  |  |
| Description | - admac_get_register_addr command processing: - Get DMAC channel N register’s address |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_015, FD_PLG_RDR_017, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-1 admac_get_register_addr flowchart

### admac_set_stage_number

DD_PLG_RDR_07_002

| Syntax | static inline void admac_set_stage_number(UWORD32 n, UWORD32 stg_num) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set stage number. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | UWORD32 | n | I | Channel number |
|  | UWORD32 | stg_num | I | Stage number |
| Return value | None |  |  |  |
| Description | - admac_set_stage_number command processing: - Set stage for DMA channel N |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-2 admac_set_stage_number flowchart

### admac_reset_descriptor

DD_PLG_RDR_07_003

| Syntax | static inline void admac_reset_descriptor(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to reset descriptor of DMA channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | None |  |  |  |
| Description | - admac_reset_descriptor command processing: - Reset descriptor of DMA channel N |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-3 admac_reset_descriptor flowchart

### admac_start_descriptor

DD_PLG_RDR_07_004

| Syntax | static inline void admac_start_descriptor(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to start descriptor of DMA channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | None |  |  |  |
| Description | - admac_start_descriptor command processing: - Start descriptor of DMA channel N |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-4 admac_start_desciptor flowchart

### admac_enable_ch

DD_PLG_RDR_07_005

| Syntax | static inline void admac_enable_ch(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to enable DMA channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | None |  |  |  |
| Description | - admac_enable_ch command processing: - Enable DMA channel N |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_015, FD_PLG_RDR_017, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-5 admac_enable_ch flowchart

### admac_disable_ch

DD_PLG_RDR_07_006

| Syntax | static inline void admac_disable_ch(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to disable DMA channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | None |  |  |  |
| Description | - admac_disable_ch command processing: - Disable DMA channel N |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_015, FD_PLG_RDR_017, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-6 admac_disable_ch flowchart

### admac_enable_module

DD_PLG_RDR_07_007

| Syntax | static inline dmac_error_code admac_enable_module(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to enable DMAC module. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | DMAC_ERROR_OUT_RANGE |  | DMAC module is invalid |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - admac_enable_module command processing: - Set ‘1’ to DE bit of register XF_RCAR_DMAOR_L or XF_RCAR_DMAOR_U depending channel number to enable DMAC module |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_015, FD_PLG_RDR_017, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-7 admac_enable_module flowchart

### admac_clear_channel

DD_PLG_RDR_07_008

| Syntax | static inline dmac_error_code admac_clear_module(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to clear value set in DMAC channel N registers. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | DMAC_ERROR_OUT_RANGE |  | DMAC module is invalid |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - admac_clear_channel command processing: - Clear value set in DMAC channel N registers |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-8 admac_clear_channel flowchart

### admac_check_stage

DD_PLG_RDR_07_009

| Syntax | inline WORD32 admac_check_stage(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to check the current stage of DMAC module. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | The stage of ADMAC module |  |  |  |
| Description | - admac_check_stage command processing: - Check the current stage of DMAC module |  |  |  |

[Covers: FD_PLG_RDR_015, FD_PLG_RDR_017]

Figure 3-9 admac_check_stage flowchart

### admac_check_and_clear_interrupt

DD_PLG_RDR_07_010

| Syntax | inline WORD32 admac_check_and_clear_interrupt(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to check and clear interrupt of DMAC channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | The interrupt_flag |  |  |  |
| Description | - admac_check_and_clear_interrupt command processing: - Check and clear the interrupt of DMAC channel N |  |  |  |

[Covers: FD_PLG_RDR_015, FD_PLG_RDR_017]

Figure 3-10 admac_check_and_clear_interrupt flowchart

### admac_start

DD_PLG_RDR_07_011

| Syntax | inline dmac_error_code admac_start(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to start DMAC channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | DMAC_ERROR_OUT_RANGE |  | DMAC module is invalid |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - admac_start command processing: - Start the DMAC channel N |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_015, FD_PLG_RDR_017, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-11admac_start flowchart

### admac_stop

DD_PLG_RDR_07_012

| Syntax | inline dmac_error_code admac_stop(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to stop DMAC channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | DMAC_ERROR_OUT_RANGE |  | DMAC module is invalid |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - admac_stop command processing: - Stop the DMAC channel N and clear value set on all of its registers |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-12 admac_stop flowchart

### admac_descriptor_setup

DD_PLG_RDR_07_013

| Syntax | static inline VOID admac_descriptor_setup(ADMA_CH n, const void *descript_mem) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set DMAC channel N’s descriptor. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
|  | const void * | descript_mem | I | Pointer to descriptor memory |
| Return value | None |  |  |  |
| Description | - admac_descriptor_setup command processing: - Set DMAC channel N’s descriptor |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-13 admac_descriptor_setup flowchart

### admac_stage_config

DD_PLG_RDR_07_014

| Syntax | static inline VOID admac_stage_config(const void *src, const void *dst, const void *stage_mem, UWORD32 trans_num) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set DMAC stage. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | const void * | src | I | Source address |
|  | const void * | dst | I | Destination address |
|  | const void * | stage_mem | I | Stage memory |
|  | UWORD32 | trans_num | I | Transmission number in a time |
| Return value | None |  |  |  |
| Description | - admac_stage_config command processing: - Set DMAC stage |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-14 admac_stage_config flowchart

### admac_pre_config

DD_PLG_RDR_07_015

| Syntax | static inline dmac_error_code admac_pre_config(ADMA_CH dma_channel, UWORD32 src_reqst, UWORD32 reqst_flag, const void *descript_mem, UWORD32 stg_num, ADMAC_MODE mode, UWORD32 transfer_size) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function | This function is to set DMAC parameters. |  |  |  |  |
| Arguments | Type | Name | I/O |  | Description |
|  | ADMA_CH | dma_channel | I |  | DMAC channel Valid range: [0: 31] |
|  | UWORD32 | src_reqst | I |  | Source request |
|  | UWORD32 | reqst_flag | I |  | Source or destination is requested Valid values: DMAC_RECEIVE_REQ DMAC_TRANSMIT_REQ |
|  | const void * | descript_mem | I |  | Descriptor memory address |
|  | UWORD32 | stg_num | I |  | Stage number Valid values: greater than zero |
|  | ADMAC_MODE | mode | I |  | Operating mode |
|  | UWORD32 | transfer_size | I |  | Transmission unit Valid values: [4] |
| Return value | DMAC_ERROR_OUT_RANGE |  |  | DMAC module is invalid |  |
|  | DMAC_ERROR_INVALID |  |  | Stage number is invalid Request address is invalid Transfer size is invalid |  |
|  | DMAC_ERROR_NONE |  |  | Normal end |  |
| Description | - admac_pre_config command processing: - Set DMAC parameters |  |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-15 admac_pre_config flowchart

### admac_post_config

DD_PLG_RDR_07_016

| Syntax | static inline VOID admac_post_config(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to post set DMAC parameters for DMAC channel N |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | None |  |  |  |
| Description | - admac_post_config command processing: - Reset the descriptor and then start it |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-16 admac_post_config flowchart

### admac_check_valid_channel

DD_PLG_RDR_07_017

| Syntax | inline dmac_error_code admac_check_valid_channel(pVOID pch) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to check availability of DMACs and choose an available one for operation. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | pVOID | pch | I/O | Pointer to channel of DMAC Valid range: [0: 31] |
| Return value | DMAC_ERROR_OUT_RANGE |  | DMAC module is invalid |  |
|  | DMAC_ERROR_BUSY |  | No available DMAC module |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - admac_check_valid_channel command processing: - Check availability of DMACs - Choose an available one for operation |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-17 admac_check_valid_channel flowchart

### admac_setting_use_descriptor

DD_PLG_RDR_07_018

| Syntax | inline dmac_error_code admac_setting_use_descriptor(const DMAC_SETTING *dmac_set) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set DMAC module using descriptor memory. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | const DMAC_SETTING * | dmac_set | I | Pointer to DMAC parameter structure |
| Return value | Error returned from function admac_pre_config if failed |  |  |  |
| Description | - admac_setting_use_descriptor command processing: - Pre set up DMAC module - Set up all stages based on transmission mode - Post set up DMAC module |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-18 admac_setting_use_descriptor flowchart

### admac_setting_nouse_descriptor

DD_PLG_RDR_07_019

| Syntax | inline dmac_error_code admac_setting_nouse_descriptor(const DMAC_SETTING *dmac_set) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set DMAC module without using descriptor memory. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | const DMAC_SETTING * | dmac_set | I | Pointer to DMAC parameter structure |
| Return value | DMAC_ERROR_OUT_RANGE |  | Cannot clear value in DMAC’s registers Cannot enable DMAC module |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - admac_setting_nouse_descriptor command processing: - Set necessary registers for ADMAC not using descriptor memory |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-19 admac_setting_nouse_descriptor flowchart

### admac_resetup

DD_PLG_RDR_07_020

| Syntax | inline dmac_error_code admac_resetup(DMAC_SETTING dmac_set)s |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function | This function is to re-setup DMAC module. |  |  |  |  |
| Arguments | Type | Name | I/O |  | Description |
|  | DMAC_SETTING | dmac_set | I |  | DMAC parameter structure |
| Return value | DMAC_ERROR_OUT_RANGE |  |  | Cannot enable DMAC module |  |
|  | DMAC_ERROR_NONE |  |  | Normal end |  |
| Description | - admac_resetup command processing: - Re-setup DMAC module |  |  |  |  |

[Covers: FD_PLG_RDR_015, FD_PLG_RDR_017]

Figure 3-20 admac_resetup flowchart

### admac_pause

DD_PLG_RDR_07_021

| Syntax | VOID admac_pause(ADMA_CH n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to pause DMAC channel N |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | ADMA_CH | n | I | Channel number Valid range: [0: 31] |
| Return value | None |  |  |  |
| Description | - admac_pause command processing: - Disable DMAC channel N |  |  |  |

[Covers: FD_PLG_RDR_014, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-21 admac_pause flowchart

# Revision history

| Version | Date | Page | Content | Approved | Changed |
| --- | --- | --- | --- | --- | --- |
| 1.0.0 | Nov 14 2018 | - | First Edition issued | Vu Phan | Vu Phan |
| 1.1.0 | Dec 10 2018 | - | Add traceability ID | Vu Phan | Ngu Pham |
| 1.2.0 | Jan 03, 2019 | - | Add range for input parameters | Vu Phan | Tien Tran |
| 1.3.0 | Mar 20, 2019 | 33, 34 | Modify transfer size correct to swap channel issue |  | Ngu Pham |