Document Type: Detail Design

Document Name:

ADSP FRAMEWORK: FIFO DRIVER

Renesas Electronics Corporation

Renesas Design Vietnam Co. Ltd

R-Car Software Solution 2 Group

Middleware 1 Team

Table of Contents

1.	Overview	3

2.	Function list	4

3.	Detail information	5

3.1	Data type definition	5

3.1.1	fifo_modules	5

3.1.2	fifo_error_code	5

3.1.3	fifo_type	5

3.1.4	FIFO_PARAM	6

3.2	Macro definition	7

3.3	Register definition	8

3.4	Function definition	10

3.4.1	fifo_disable	10

3.4.2	fifo_enable	12

3.4.3	fifo_config	14

3.4.4	fifo_get_int_num	18

3.4.5	fifo_register_interrupt_handler	19

3.4.6	fifo_unregister_interrupt_handler	21

3.4.7	fifo_disable_interrupt	23

3.4.8	fifo_enable_interrupt	25

3.4.9	fifo_clear_interrupt	27

3.4.10	fifo_stop	29

3.4.11	fifo_check_available	31

4.	Revision history	33

List of Figures

Figure 1-1 The software architecture	3

Figure 3-1 fifo_disable flowchart	11

Figure 3-2 fifo_enable flowchart	13

Figure 3-3 fifo_config flowchart	17

Figure 3-4 fifo_get_int_num flowchart	18

Figure 3-5 fifo_register_interrupt_handler flowchart	20

Figure 3-6 fifo_unregister_interrupt_handler flowchart	22

Figure 3-7 fifo_disable_interrupt flowchart	24

Figure 3-8 fifo_enable_interrupt flowchart	26

Figure 3-9 fifo_clear_interrupt flowchart	28

Figure 3-10 fifo_stop flowchart	30

Figure 3-11 fifo_check_available flowchart	32

List of Table

Table 2-1 Function list	4

Table 3-1 fifo_modules type information	5

Table 3-2 fifo_error_code type information	5

Table 3-3 fifo_type type information	5

Table 3-4 FIFO_PARAM type information	6

Table 3-5 Setting registers OF_EN and IF_EN to disable FIFO	10

Table 3-6 Setting registers OF_EN and IF_EN to enable FIFO	12

Table 2-1 Function list	4

Table 3-1 fifo_modules type information	5

Table 3-2 fifo_error_code type information	5

Table 3-3 fifo_type type information	5

Table 3-4 FIFO_PARAM type information	6

Table 3-5 Setting registers OF_EN and IF_EN to disable FIFO	10

Table 3-6 Setting registers OF_EN and IF_EN to enable FIFO	12

# Overview

This section provides an overview of software architecture.

User Application

ADSP Interface

Kernel Space

User Space

ADSP Driver

ARM

Audio HW

ADSP Framework

TDM class

Renderer class

Equalizer class

Capture class

TDM Plugin

ADSP

Plugin

Equalizer Plugin*

Capture Plugin

Renderer Plugin

This document’s target is in side of red square

* not connect to hardware modules

- DAC/

- ADC

FIFO

SCU

PDMA

SSI

SSIU

ADMA

Figure 1-1 The software architecture

# Function list

The following is list of functions:

Table 2-1 Function list

| Type | Function Name | Outline |
| --- | --- | --- |
| API | fifo_enable | This API is to enable the input/output of FIFO |
|  | fifo_disable | This API is to disable the input/output of FIFO |
|  | fifo_config | This API is to set up registers necessary for FIFO module execution |
|  | fifo_register_interrupt_handler | This API is to register interrupt handler for FIFO |
|  | fifo_unregister_interrupt_handler | This API is to unregister interrupt handler for FIFO |
|  | fifo_disable_interrupt | This API is to disable FIFO interrupt |
|  | fifo_enable_interrupt | This API is to enable FIFO interrupt |
|  | fifo_clear_interrupt | This API is to clear FIFO interrupt |
|  | fifo_stop | This API is to stop FIFO operation |
|  | fifo_check_available | This API is to check availability of FIFO module and choose an available one |
| Internal function | fifo_get_int_num | This function is to get the interrupt number |

# Detail information

This section describes detail information of data types, macro definitions, implemented APIs and internal function units, global variable.

## Data type definition

### fifo_modules

The data type fifo_modules is a type-defined enumeration that lists all supported FIFO modules.

Table 3-1 fifo_modules type information

| Member name | Value | Outline |
| --- | --- | --- |
| FIFO0 | 0 | 1st FIFO |
| FIFO1 | 1 | 2nd FIFO |
| FIFO2 | 2 | 3rd FIFO |
| FIFO3 | 3 | 4th FIFO |
| FIFOMAX | 4 | Number of FIFO modules |

### fifo_error_code

The data type fifo_error_code is a type-defined enumeration that lists all error codes for FIFO.

Table 3-2 fifo_error_code type information

| Member name | Value |
| --- | --- |
| FIFO_ERROR_NONE | 0 |
| FIFO_ERROR_INVALID | -1 |
| FIFO_ERROR_INTERNAL | -2 |
| FIFO_ERROR_BUSY | -3 |
| FIFO_ERROR_OUT_RANGE | -4 |

### fifo_type

The data type fifo_type is a type-defined enumeration of indicators of whether FIFO’s input or output is used.

Table 3-3 fifo_type type information

| Member name | Value | Outline |
| --- | --- | --- |
| FIFO_INPUT | 0 | FIFO input is used |
| FIFO_OUTPUT | 1 | FIFO output is used |

### FIFO_PARAM

The data type FIFO_PARAM is a type-defined structure that possesses necessary parameters for FIFO module.

Table 3-4 FIFO_PARAM type information

| Member name | Data type | Outline |
| --- | --- | --- |
| channel | UWORD32 | channel of data used for FIFO module |
| pcm_width | UWORD32 | PCM width of data used for FIFO module |
| sample_rate | UWORD32 | Sampling rate of data used for FIFO module |
| fifo_num | fifo_modules | FIFO module |
| type | fifo_type | FIFO type |
| dev_connect | UWORD32 | Indicator to show which module FIFO connects |

## Macro definition

| Macros | Value | Outline |
| --- | --- | --- |
| XA_FIFO_FULL | 1024 | FIFO full size |
| XA_FIFO_NEARFULL | 768 | FIFO nearly full size |
| XA_FIFO_NEAREMPTY | 64 | FIFO nearly empty size |
| XA_FIFO_EMPTY | 0 | FIFO empty size |
| XA_FIFO_CONNECT_SRC | 0 | FIFO is connected to SRC |
| XA_FIFO_CONNECT_SSI | 1 | FIFO is connected to SSI |

## Register definition

Below is the table listing all items that are macros representing register addresses located in repository s492d/include/sys/xt-shmem/board-rcar/xf-registers.h

| Register | Outline |
| --- | --- |
| XF_RCAR_REG_OF_EN | Register OF_EN |
| XF_RCAR_REG_IF_EN | Register IF_EN |
| XF_RCAR_REG_ADSP_TIMSEL | ADSP Output Timing Select Register |
| XF_RCAR_REG_ADSP_TIMSEL1 | ADSP Output Timing Select Register 1 |
| XF_RCAR_REG_ADSP_TIMSEL2 | ADSP Output Timing Select Register 2 |
| XF_RCAR_REG_ADSP_TIMSEL3 | ADSP Output Timing Select Register 3 |
| XF_RCAR_REG_OF_MSTP | Output FIFO Module Stop Register |
| XF_RCAR_REG_OF_PTRRST | Output FIFO Pointer Reset Register |
| XF_RCAR_REG_OF0_CH | Output FIFO0 Channel Register |
| XF_RCAR_REG_OF1_CH | Output FIFO1 Channel Register |
| XF_RCAR_REG_OF2_CH | Output FIFO2 Channel Register |
| XF_RCAR_REG_OF3_CH | Output FIFO3 Channel Register |
| XF_RCAR_REG_OF_ADINR | Output FIFO0 Audio Information Register |
| XF_RCAR_REG_OF1_ADINR | Output FIFO1 Audio Information Register |
| XF_RCAR_REG_OF2_ADINR | Output FIFO2 Audio Information Register |
| XF_RCAR_REG_OF3_ADINR | Output FIFO3 Audio Information Register |
| XF_RCAR_REG_OF0_INT_TH | Output FIFO0 Interrupt Threshold Register |
| XF_RCAR_REG_OF1_INT_TH | Output FIFO1 Interrupt Threshold Register |
| XF_RCAR_REG_OF2_INT_TH | Output FIFO2 Interrupt Threshold Register |
| XF_RCAR_REG_OF3_INT_TH | Output FIFO3 Interrupt Threshold Register |
| XF_RCAR_REG_OF_INT_CLR | Output FIFO Interrupt Clear Register |
| XF_RCAR_REG_IF_MSTP | Input FIFO Module Stop Register |
| XF_RCAR_REG_IF_PTRRST | Input FIFO Pointer Reset Register |
| XF_RCAR_REG_IF0_CH | Input FIFO0 Channel Register |
| XF_RCAR_REG_IF1_CH | Input FIFO1 Channel Register |
| XF_RCAR_REG_IF2_CH | Input FIFO2 Channel Register |
| XF_RCAR_REG_IF3_CH | Input FIFO3 Channel Register |
| XF_RCAR_REG_IF_ADINR | Input FIFO0 Audio Information Register |
| XF_RCAR_REG_IF1_ADINR | Input FIFO1 Audio Information Register |
| XF_RCAR_REG_IF2_ADINR | Input FIFO2 Audio Information Register |
| XF_RCAR_REG_IF3_ADINR | Input FIFO3 Audio Information Register |
| XF_RCAR_REG_IF0_INT_TH | Input FIFO0 Interrupt Threshold Register |
| XF_RCAR_REG_IF1_INT_TH | Input FIFO1 Interrupt Threshold Register |
| XF_RCAR_REG_IF2_INT_TH | Input FIFO2 Interrupt Threshold Register |
| XF_RCAR_REG_IF3_INT_TH | Input FIFO3 Interrupt Threshold Register |
| XF_RCAR_REG_IF_INT_CLR | Input FIFO Interrupt Clear Register |

## Function definition

### fifo_disable

DD_PLG_RDR_06_001

| Syntax | inline fifo_error_code fifo_disable(fifo_modules fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to disable the input/output of FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | fifo_modules | fifo_num | I | FIFO module Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO uses its input or output Valid values: FIFO_OUTPUT FIFO_INPUT |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type is neither FIFO_INPUT nor FIFO_OUTPUT |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_disable command processing: - Set either register OF_EN or IF_EN based on FIFO type |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_032, FD_PLG_RDR_043]

Table 3-5 Setting registers OF_EN and IF_EN to disable FIFO

| fifo_num | type | Register | Bit | Value |
| --- | --- | --- | --- | --- |
| FIFO0 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 0 | 0 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 0 | 0 |
| FIFO1 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 1 | 0 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 1 | 0 |
| FIFO2 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 2 | 0 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 2 | 0 |
| FIFO3 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 3 | 0 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 3 | 0 |

Figure 3-1 fifo_disable flowchart

### fifo_enable

DD_PLG_RDR_06_002

| Syntax | inline fifo_error_code fifo_enable(fifo_modules fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to enable the input/output of FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | fifo_modules | fifo_num | I | FIFO module Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO uses its input or output Valid values: FIFO_OUTPUT FIFO_INPUT |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type is neither FIFO_INPUT nor FIFO_OUTPUT |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_enable command processing: - Set either register OF_EN or IF_EN based on FIFO type |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_032, FD_PLG_RDR_043]

Table 3-6 Setting registers OF_EN and IF_EN to enable FIFO

| fifo_num | type | Register | Bit | Value |
| --- | --- | --- | --- | --- |
| FIFO0 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 0 | 1 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 0 | 1 |
| FIFO1 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 1 | 1 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 1 | 1 |
| FIFO2 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 2 | 1 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 2 | 1 |
| FIFO3 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 3 | 1 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 3 | 1 |

Figure 3-2 fifo_enable flowchart

### fifo_config

DD_PLG_RDR_06_003

| Syntax | inline fifo_error_code fifo_config(FIFO_PARAM fifo_param) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set up registers necessary for FIFO module execution. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_PARAM | fifo_param | I | Struct of parameters of FIFO |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type is invalid Channel is invalid PCM width is invalid Sampling rate is invalid when FIFO type is FIFO_OUTPUT FIFO is not connected to and hardware module when FIFO type is FIFO_OUTPUT |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_config command processing: - Set registers SSI_BUSIF_MODE, SSI_BUSIF_ADINR, SSI mode register |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-3 fifo_config flowchart

### fifo_get_int_num

DD_PLG_RDR_06_004

| Syntax | static inline fifo_error_code fifo_get_int_num(fifo_modules module, fifo_type type, pUWORD32 num) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to get the interrupt number of FIFO module. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | fifo_modules | fifo_num | I | FIFO module Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: FIFO_OUTPUT FIFO_INPUT |
|  | pUWORD32 | num | I/O | Interrupt number |
| Return value | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_get_int_num command processing: - Get interrupt number based on FIFO module and its type (FIFO_INPUT or FIFO_OUTPUT) |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-4 fifo_get_int_num flowchart

### fifo_register_interrupt_handler

DD_PLG_RDR_06_005

| Syntax | inline fifo_error_code fifo_register_interrupt_handler(fifo_modules fifo_num, fifo_type type, pVOID function_addr, pVOID arg) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to register interrupt handler for FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | fifo_modules | fifo_num | I | FIFO module Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: FIFO_OUTPUT FIFO_INPUT |
|  | pVOID | function_addr | I | Pointer to handler function |
|  | pVOID | arg | I | Argument passed to handler |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type or function_addr is invalid |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_register_interrupt_handler command processing: - Register interrupt handler for FIFO |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-5 fifo_register_interrupt_handler flowchart

### fifo_unregister_interrupt_handler

DD_PLG_RDR_06_006

| Syntax | inline fifo_error_code fifo_unregister_interrupt_handler(fifo_modules fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to unregister interrupt handler for FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | fifo_modules | fifo_num | I | FIFO module Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: FIFO_OUTPUT FIFO_INPUT |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type is invalid |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_unregister_interrupt_handler command processing: - Unregister interrupt handler for FIFO |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-6 fifo_unregister_interrupt_handler flowchart

### fifo_disable_interrupt

DD_PLG_RDR_06_007

| Syntax | inline fifo_error_code fifo_disable_interrupt(fifo_modules fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to disable interrupt for FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | fifo_modules | fifo_num | I | FIFO module Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: FIFO_OUTPUT FIFO_INPUT |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type is invalid |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_disable_interrupt command processing: - Disable FIFO interrupt |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-7 fifo_disable_interrupt flowchart

### fifo_enable_interrupt

DD_PLG_RDR_06_008

| Syntax | inline fifo_error_code fifo_enable_interrupt(fifo_modules fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to enable interrupt for FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | fifo_modules | fifo_num | I | FIFO module Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: FIFO_OUTPUT FIFO_INPUT |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type is invalid |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_enable_interrupt command processing: - Enable FIFO interrupt |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-8 fifo_enable_interrupt flowchart

### fifo_clear_interrupt

DD_PLG_RDR_06_009

| Syntax | inline fifo_error_code fifo_clear_interrupt(fifo_modules fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to clear FIFO interrupt. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | fifo_modules | fifo_num | I | FIFO module Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: FIFO_OUTPUT FIFO_INPUT |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type is invalid |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_clear_interrupt command processing: - Clear FIFO interrupt |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-9 fifo_clear_interrupt flowchart

### fifo_stop

DD_PLG_RDR_06_010

| Syntax | inline fifo_error_code fifo_stop(fifo_modules fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to stop FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | fifo_modules | fifo_num | I | FIFO module Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: FIFO_OUTPUT FIFO_INPUT |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type is invalid |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_stop command processing: - Stop FIFO operation |  |  |  |

[Covers: FD_PLG_RDR_005, FD_PLG_RDR_014, FD_PLG_RDR_032, FD_PLG_RDR_043]

Figure 3-10 fifo_stop flowchart

### fifo_check_available

DD_PLG_RDR_06_011

| Syntax | inline fifo_error_code fifo_check_available(UWORD32 *fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to check availability of FIFOs and choose an available one for operation. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | UWORD32 * | fifo_num | I/O | Pointer to FIFO number Valid range: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: FIFO_OUTPUT FIFO_INPUT |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_INVALID |  | FIFO type is invalid |  |
|  | FIFO_ERROR_BUSY |  | No available FIFO module |  |
|  | FIFO_ERROR_NONE |  | Normal end |  |
| Description | - fifo_check_available command processing: - Check availability of FIFOs - Choose an available one for operation |  |  |  |

[Covers: FD_PLG_RDR_005]

Figure 3-11 fifo_check_available flowchart

# Revision history

| Version | Date | Page | Content | Approved | Changed |
| --- | --- | --- | --- | --- | --- |
| 1.0.0 | Nov 14 2018 | - | First Edition issued | Vu Phan | Vu Phan |
| 1.1.0 | Dec 10 2018 | - | Add traceability ID | Vu Phan | Ngu Pham |
| 1.2.0 | Jan 03, 2019 | - | Add range for input parameters | Vu Phan | Tien Tran |