Document Type: Detail Design

Document Name:

ADSP FRAMEWORK: ADMA DRIVER

Renesas Electronics Corporation

Renesas Design Vietnam Co. Ltd

R-Car Software Solution 2 Group

Middleware 1 Team

Table of Contents

1.	Overview	3

2.	Function list	4

3.	Detail information	5

3.1	Data type definition	5

3.1.1	DMAC_CHN	5

3.1.2	DMAC_ERROR_CODE	6

3.1.3	DMAC_SRC_DST	6

3.1.4	DMAC_MODE	10

3.1.5	DMAC_PARAMS	10

3.1.6	DMAC_MODULE_INFO	11

3.2	Macro definition	12

3.3	Register definition	14

3.4	Global variable definition	15

3.5	Function definition	19

3.5.1	xa_dmac_get_register_addr	19

3.5.2	xa_dmac_set_stage_number	20

3.5.3	xa_dmac_reset_descriptor	21

3.5.4	xa_dmac_start_descriptor	22

3.5.5	xa_dmac_enable_ch	23

3.5.6	xa_dmac_disable_ch	24

3.5.7	xa_dmac_enable_module	25

3.5.8	xa_dmac_clear_channel	26

3.5.9	xa_dmac_check_stage	27

3.5.10	xa_dmac_check_and_clear_interrupt	28

3.5.11	xa_dmac_start	29

3.5.12	xa_dmac_stop	30

3.5.13	xa_dmac_descriptor_setup	31

3.5.14	xa_dmac_stage_config	32

3.5.15	xa_dmac_pre_config	33

3.5.16	xa_dmac_post_config	35

3.5.17	xa_dmac_check_valid_channel	36

3.5.18	xa_dmac_setting_use_descriptor	38

3.5.19	xa_dmac_setting_nouse_descriptor	40

3.5.20	xa_dmac_resetup	42

4.	Revision history	43

List of Figures

Figure 1-1 The software architecture	3

Figure 3-1 xa_dmac_get_register_addr flowchart	19

Figure 3-2 xa_dmac_set_stage_number flowchart	20

Figure 3-3 xa_dmac_reset_descriptor flowchart	21

Figure 3-4 xa_dmac_start_descriptor flowchart	22

Figure 3-5 xa_dmac_enable_ch flowchart	23

Figure 3-6 xa_dmac_disable_ch flowchart	24

Figure 3-7 xa_dmac_enable_module flowchart	25

Figure 3-8 xa_dmac_clear_channel flowchart	26

Figure 3-9 xa_dmac_check_stage flowchart	27

Figure 3-10 xa_dmac_check_and_clear_interrupt flowchart	28

Figure 3-11 xa_dmac_start flowchart	29

Figure 3-12 xa_dmac_stop flowchart	30

Figure 3-13 xa_dmac_descriptor_setup flowchart	31

Figure 3-14 xa_dmac_stage_config flowchart	32

Figure 3-15 xa_dmac_pre_config flowchart	34

Figure 3-16 xa_dmac_post_config flowchart	35

Figure 3-17 xa_dmac_check_valid_channel flowchart	37

Figure 3-18 xa_dmac_setting_use_descriptor flowchart	39

Figure 3-19 xa_dmac_setting_nouse_descriptor flowchart	41

Figure 3-20 xa_dmac_resetup flowchart	42

List of Table

Table 2-1 Function list	4

Table 3-1 DMAC_CHN type information	5

Table 3-2 DMAC_ERROR_CODE type information	6

Table 3-3 DMAC_SRC_DST type information	6

Table 3-4 DMAC_MODE type information	10

Table 3-5 DMAC_PARAMS type information	10

Table 3-6 DMAC_MODULE_INFO type information	11

Table 2-1 Function list	4

Table 3-1 DMAC_CHN type information	5

Table 3-2 DMAC_ERROR_CODE type information	6

Table 3-3 DMAC_SRC_DST type information	6

Table 3-4 DMAC_MODE type information	10

Table 3-5 DMAC_PARAMS type information	10

Table 3-6 DMAC_MODULE_INFO type information	11

# Overview

This section provides an overview of software architecture.

User Application

ADSP Interface

Kernel Space

User Space

ADSP Driver

ARM

Audio HW

ADSP Framework

TDM class

Renderer class

Equalizer class

Capture class

TDM Plugin

ADSP

Plugin

Equalizer Plugin*

Capture Plugin

Renderer Plugin

This document’s target is in side of red square

* Not connect to hardware modules

- DAC/

- ADC

ADMAC

FIFO

SCU

PDMA

SSI

SSIU

Figure 1-1 The software architecture

# Function list

The following is list of functions:

Table 2-1 Function list

| Type | Function Name | Outline |
| --- | --- | --- |
| API | xa_dmac_get_register_addr | This API is to get DMAC channel N register’s address |
|  | xa_dmac_check_stage | This API is to check the current stage of DMAC module |
|  | xa_dmac_check_and_clear_interrupt | This API is to check and clear interrupt of DMAC channel N |
|  | xa_dmac_start | This API is to start DMAC channel N |
|  | xa_dmac_stop | This API is to stop DMAC channel N |
|  | xa_dmac_check_valid_channel | This API is to check availability of DMAC module and choose an available one |
|  | xa_dmac_setting_use_descriptor | This API is to set DMAC module using descriptor memory |
|  | xa_dmac_setting_nouse_descriptor | This API is to set DMAC module not using descriptor memory |
|  | xa_dmac_resetup | This API is to re-setup DMAC module |
| Internal function | xa_dmac_set_stage_number | This function is to set stage number |
|  | xa_dmac_reset_descriptor | This function is to reset descriptor |
|  | xa_dmac_start_descriptor | This function is to start descriptor |
|  | xa_dmac_enable_ch | This function is to enable a channel |
|  | xa_dmac_disable_ch | This function is to disable a channel |
|  | xa_dmac_enable_module | This function is to enable DMAC module |
|  | xa_dmac_clear_channel | This function is to clear value set in DMAC channel N register |
|  | xa_dmac_descriptor_setup | This function is to set up DMAC descriptor |
|  | xa_dmac_stage_config | This function is to set DMAC stage |
|  | xa_dmac_pre_config | This function is to set DMAC parameters |
|  | xa_dmac_post_config | This function is to post-set DMAC parameters |

# Detail information

This section describes detail information of data types, macro definitions, implemented APIs and internal function units, global variable.

## Data type definition

### DMAC_CHN

The data type DMAC_CHN is a type-defined enumeration that lists all supported ADMAC channels.

Table 3-1 DMAC_CHN type information

| Member name | Value | Outline |
| --- | --- | --- |
| ADMAC_CH00 | 0 | ADMAC channel 0 |
| ADMAC_CH01 | 1 | ADMAC channel 1 |
| ADMAC_CH02 | 2 | ADMAC channel 2 |
| ADMAC_CH03 | 3 | ADMAC channel 3 |
| ADMAC_CH04 | 4 | ADMAC channel 4 |
| ADMAC_CH05 | 5 | ADMAC channel 5 |
| ADMAC_CH06 | 6 | ADMAC channel 6 |
| ADMAC_CH07 | 7 | ADMAC channel 7 |
| ADMAC_CH08 | 8 | ADMAC channel 8 |
| ADMAC_CH09 | 9 | ADMAC channel 9 |
| ADMAC_CH10 | 10 | ADMAC channel 10 |
| ADMAC_CH11 | 11 | ADMAC channel 11 |
| ADMAC_CH12 | 12 | ADMAC channel 12 |
| ADMAC_CH13 | 13 | ADMAC channel 13 |
| ADMAC_CH14 | 14 | ADMAC channel 14 |
| ADMAC_CH15 | 15 | ADMAC channel 15 |
| ADMAC_CH16 | 16 | ADMAC channel 16 |
| ADMAC_CH17 | 17 | ADMAC channel 17 |
| ADMAC_CH18 | 18 | ADMAC channel 18 |
| ADMAC_CH19 | 19 | ADMAC channel 19 |
| ADMAC_CH20 | 20 | ADMAC channel 20 |
| ADMAC_CH21 | 21 | ADMAC channel 21 |
| ADMAC_CH22 | 22 | ADMAC channel 22 |
| ADMAC_CH23 | 23 | ADMAC channel 23 |
| ADMAC_CH24 | 24 | ADMAC channel 24 |
| ADMAC_CH25 | 25 | ADMAC channel 25 |
| ADMAC_CH26 | 26 | ADMAC channel 26 |
| ADMAC_CH27 | 27 | ADMAC channel 27 |
| ADMAC_CH28 | 28 | ADMAC channel 28 |
| ADMAC_CH29 | 29 | ADMAC channel 29 |
| ADMAC_CH30 | 30 | ADMAC channel 30 |
| ADMAC_CH31 | 31 | ADMAC channel 31 |
| ADMAC_CHMAX | 32 | Total number of ADMAC channels |

### DMAC_ERROR_CODE

The data type DMAC_ERROR_CODE is a type-defined enumeration that lists all error codes for ADMAC.

Table 3-2 DMAC_ERROR_CODE type information

| Member name | Value |
| --- | --- |
| DMAC_ERROR_NONE | 0 |
| DMAC_ERROR_BUSY | -1 |
| DMAC_ERROR_INVALID | -2 |
| DMAC_ERROR_OUT_RANGE | -3 |

### DMAC_SRC_DST

The data type DMAC_SRC_DST is a type-defined enumeration that shows connections between ADMAC and other devices.

Table 3-3 DMAC_SRC_DST type information

| Member name | Value |
| --- | --- |
| DMAC_SSI00 | DMAC_SSI_MODULE_MIN |
| DMAC_SSI01 | 1 |
| DMAC_SSI02 | 2 |
| DMAC_SSI03 | 3 |
| DMAC_SSI04 | 4 |
| DMAC_SSI05 | 5 |
| DMAC_SSI06 | 6 |
| DMAC_SSI07 | 7 |
| DMAC_SSI10 | 10 |
| DMAC_SSI11 | 11 |
| DMAC_SSI12 | 12 |
| DMAC_SSI13 | 13 |
| DMAC_SSI14 | 14 |
| DMAC_SSI15 | 15 |
| DMAC_SSI16 | 16 |
| DMAC_SSI17 | 17 |
| DMAC_SSI20 | 20 |
| DMAC_SSI21 | 21 |
| DMAC_SSI22 | 22 |
| DMAC_SSI23 | 23 |
| DMAC_SSI24 | 24 |
| DMAC_SSI25 | 25 |
| DMAC_SSI26 | 26 |
| DMAC_SSI27 | 27 |
| DMAC_SSI30 | 30 |
| DMAC_SSI31 | 31 |
| DMAC_SSI32 | 32 |
| DMAC_SSI33 | 33 |
| DMAC_SSI34 | 34 |
| DMAC_SSI35 | 35 |
| DMAC_SSI36 | 36 |
| DMAC_SSI37 | 37 |
| DMAC_SSI40 | 40 |
| DMAC_SSI41 | 41 |
| DMAC_SSI42 | 42 |
| DMAC_SSI43 | 43 |
| DMAC_SSI44 | 44 |
| DMAC_SSI45 | 45 |
| DMAC_SSI46 | 46 |
| DMAC_SSI47 | 47 |
| DMAC_SSI5 | 50 |
| DMAC_SSI6 | 60 |
| DMAC_SSI7 | 70 |
| DMAC_SSI8 | 80 |
| DMAC_SSI90 | 90 |
| DMAC_SSI91 | 91 |
| DMAC_SSI92 | 92 |
| DMAC_SSI93 | 93 |
| DMAC_SSI94 | 94 |
| DMAC_SSI95 | 95 |
| DMAC_SSI96 | 96 |
| DMAC_SSI97 | DMAC_SSI_MODULE_MAX |
| DMAC_DTCPPP0 | 98 |
| DMAC_DTCPPP1 | 99 |
| DMAC_DTCPCP0 | 100 |
| DMAC_DTCPCP1 | 101 |
| DMAC_SCU_SRCI0 | DMAC_SCU_SRC_INPUT_MODULE_MIN |
| DMAC_SCU_SRCI1 | 111 |
| DMAC_SCU_SRCI2 | 112 |
| DMAC_SCU_SRCI3 | 113 |
| DMAC_SCU_SRCI4 | 114 |
| DMAC_SCU_SRCI5 | 115 |
| DMAC_SCU_SRCI6 | 116 |
| DMAC_SCU_SRCI7 | 117 |
| DMAC_SCU_SRCI8 | 118 |
| DMAC_SCU_SRCI9 | DMAC_SCU_SRC_INPUT_MODULE_MAX |
| DMAC_SCU_SRCO0 | DMAC_SCU_SRC_OUTPUT_MODULE_MIN |
| DMAC_SCU_SRCO1 | 121 |
| DMAC_SCU_SRCO2 | 122 |
| DMAC_SCU_SRCO3 | 123 |
| DMAC_SCU_SRCO4 | 124 |
| DMAC_SCU_SRCO5 | 125 |
| DMAC_SCU_SRCO6 | 126 |
| DMAC_SCU_SRCO7 | 127 |
| DMAC_SCU_SRCO8 | 128 |
| DMAC_SCU_SRCO9 | DMAC_SCU_SRC_OUTPUT_MODULE_MAX |
| DMAC_SCU_CMD0 | DMAC_SCU_CMD_MODULE_MIN |
| DMAC_SCU_CMD1 | DMAC_SCU_CMD_MODULE_MAX |
| DMAC_MLM0 | 132 |
| DMAC_MLM1 | 133 |
| DMAC_MLM2 | 134 |
| DMAC_MLM3 | 135 |
| DMAC_MLM4 | 136 |
| DMAC_MLM5 | 137 |
| DMAC_MLM6 | 138 |
| DMAC_MLM7 | 139 |
| DMAC_NONCONTROL | 140 |
| DMAC_MAX | 141 |

### DMAC_MODE

The data type DMAC_MODE is a type-defined enumeration of ADMAC operation mode.

Table 3-4 DMAC_MODE type information

| Member name | Value | Outline |
| --- | --- | --- |
| ADMAC_NORMAL_MODE | 1 | ADMAC normal mode |
| ADMAC_REPEAT_MODE | 2 | ADMAC repeating mode |

### DMAC_PARAMS

The data type DMAC_PARAMS is a type-defined structure that possesses necessary parameters for DMAC module.

Table 3-5 DMAC_PARAMS type information

| Member name | Data type | Outline |
| --- | --- | --- |
| dst | WORD32 | Destination address |
| src | WORD32 | Source address |
| connect_flag | WORD32 | Transmission type flag |
| transmem | pVOID | Transmission memory address |
| trans_num | WORD32 | Transfer number |
| stg_num | WORD32 | Stage number |
| stg_size | WORD32 | Stage memory size |
| buf_size | WORD32 | Ring buffer size |
| trans_mode | WORD32 | Indicator of transfer/receive mode |
| mode | DMAC_MODE | Operation mode |
| dma_ch | UWORD32 | ADMAC channel |
| src_reqst | UWORD32 | Source request |
| trans_size | UWORD32 | Transfer size |

### DMAC_MODULE_INFO

The data type DMAC_MODULE_INFO is a type-defined structure of ADMAC device infomation.

Table 3-6 DMAC_MODULE_INFO type information

| Member name | Data type | Outline |
| --- | --- | --- |
| module_address | UWORD32 | Destination address |
| trans | UWORD32 | Source address |
| recei | UWORD32 | Transmission memory address |

## Macro definition

| Macros | Value | Outline |
| --- | --- | --- |
| DMAC_SSI_MODULE_MIN | 0 | SSI module min index |
| DMAC_SSI_MODULE_MAX | 97 | SSI module max index |
| DMAC_SCU_SRC_INPUT_MODULE_MIN | 110 | SCU input module min index |
| DMAC_SCU_SRC_INPUT_MODULE_MAX | 119 | SCU input module max index |
| DMAC_SCU_SRC_OUTPUT_MODULE_MIN | 120 | SCU output module min index |
| DMAC_SCU_SRC_OUTPUT_MODULE_MAX | 129 | SCU output module max index |
| DMAC_SCU_CMD_MODULE_MIN | 130 | CMD module min index |
| DMAC_SCU_CMD_MODULE_MAX | 131 | CMD module max index |
| XF_RCAR_DMAOR_L | XF_RCAR_REG_DMAOR_L | DMA operation register (for lower-numbered channels) |
| XF_RCAR_DMAOR_U | XF_RCAR_REG_DMAOR_U | DMA channel clear register (for lower-numbered channels) |
| XF_DMACHCLR_L | XF_RCAR_REG_DMACHCLR_L | DMA channel clear register (for lower-numbered channels) |
| XF_DMACHCLR_U | XF_RCAR_REG_DMACHCLR_U | DMA channel clear register (for higher-numbered channels) |
| XF_RCAR_DMARS_0 | XF_RCAR_REG_DMARS_0 | Channel 0 DMA Extended Resource Selectors Register |
| XF_RCAR_DMADPBASE_0 | XF_RCAR_REG_DMADPBASE_0 | Channel 0 DMA Descriptor Base Address Register |
| XF_RCAR_DMACHCRB_0 | XF_RCAR_REG_DMACHCRB_0 | Channel 0 DMA channel control register B |
| XF_RCAR_DMACHCR_0 | XF_RCAR_REG_DMACHCR_0 | Channel 0 DMA Channel Control Register |
| XF_RCAR_DMATCR_0 | XF_RCAR_REG_DMATCR_0 | Channel 0 DMA Transfer Count Register |
| XF_RCAR_DMABUFCR_0 | XF_RCAR_REG_DMABUFCR_0 | Channel 0 DMA Buffer Control register |
| XF_RCAR_DMASAR_0 | XF_RCAR_REG_DMASAR_0 | Channel 0 Source Address register |
| XF_RCAR_DMADAR_0 | XF_RCAR_REG_DMADAR_0 | Channel 0 Destination Address register |
| DMAC_TRANSMIT_REQ | 1 | source request is source |
| DMAC_RECEIVE_REQ | 2 | source request is destination |

## Register definition

Below is the table listing all items that are macros representing register addresses located in repository s492d/include/sys/xt-shmem/board-rcar/xf-registers.h

| Register | Outline |
| --- | --- |
| XF_RCAR_REG_DMAOR_L | DMA operation register (for lower-numbered channels) |
| XF_RCAR_REG_DMAOR_U | DMA channel clear register (for lower-numbered channels) |
| XF_RCAR_REG_DMACHCLR_L | DMA channel clear register (for lower-numbered channels) |
| XF_RCAR_REG_DMACHCLR_U | DMA channel clear register (for higher-numbered channels) |
| XF_RCAR_REG_DMARS_0 | Channel 0 DMA Extended Resource Selectors Register |
| XF_RCAR_REG_DMADPBASE_0 | Channel 0 DMA Descriptor Base Address Register |
| XF_RCAR_REG_DMACHCRB_0 | Channel 0 DMA channel control register B |
| XF_RCAR_REG_DMACHCR_0 | Channel 0 DMA Channel Control Register |
| XF_RCAR_REG_DMATCR_0 | Channel 0 DMA Transfer Count Register |
| XF_RCAR_REG_DMABUFCR_0 | Channel 0 DMA Buffer Control register |
| XF_RCAR_REG_DMASAR_0 | Channel 0 Source Address register |
| XF_RCAR_REG_DMADAR_0 | Channel 0 Destination Address register |

## Global variable definition

Below is the array of struct DMAC_MODULE_INFO-datatype admac_set_value holds 141 (DMAC_MAX) elements representing settings for ADMAC data transfer

| static DMAC_MODULE_INFO admac_set_value[DMAC_MAX] |  |  |  |
| --- | --- | --- | --- |
| Array’s index | module_address | trans | recei |
| DMAC_SSI00 | 0xEC100000 | 0x15 | 0x16 |
| DMAC_SSI01 | 0xEC100400 | 0x35 | 0x36 |
| DMAC_SSI02 | 0xEC100800 | 0x37 | 0x38 |
| DMAC_SSI03 | 0xEC100C00 | 0x47 | 0x48 |
| DMAC_SSI04 | 0xEC10A000 | 0x3F | 0x40 |
| DMAC_SSI05 | 0xEC10A400 | 0x43 | 0x44 |
| DMAC_SSI06 | 0xEC10A800 | 0x4F | 0x50 |
| DMAC_SSI07 | 0xEC10AC00 | 0x53 | 0x54 |
| DMAC_SSI10 | 0xEC101000 | 0x49 | 0x4A |
| DMAC_SSI11 | 0xEC101400 | 0x4B | 0x4C |
| DMAC_SSI12 | 0xEC101800 | 0x57 | 0x58 |
| DMAC_SSI13 | 0xEC101C00 | 0x59 | 0x5A |
| DMAC_SSI14 | 0xEC10B000 | 0x5F | 0x60 |
| DMAC_SSI15 | 0xEC10B400 | 0xC3 | 0xC4 |
| DMAC_SSI16 | 0xEC10B800 | 0xC7 | 0xC8 |
| DMAC_SSI17 | 0xEC10BC00 | 0xCB | 0xCC |
| DMAC_SSI20 | 0xEC102000 | 0x63 | 0x64 |
| DMAC_SSI21 | 0xEC102400 | 0x67 | 0x68 |
| DMAC_SSI22 | 0xEC102800 | 0x6B | 0x6C |
| DMAC_SSI23 | 0xEC102C00 | 0x6D | 0x6E |
| DMAC_SSI24 | 0xEC10C000 | 0xCF | 0xCE |
| DMAC_SSI25 | 0xEC10C400 | 0xEB | 0xEC |
| DMAC_SSI26 | 0xEC10C800 | 0xED | 0xEE |
| DMAC_SSI27 | 0xEC10CC00 | 0xEF | 0xF0 |
| DMAC_SSI30 | 0xEC103000 | 0x6F | 0x70 |
| DMAC_SSI31 | 0xEC103400 | 0x21 | 0x22 |
| DMAC_SSI32 | 0xEC103800 | 0x23 | 0x24 |
| DMAC_SSI33 | 0xEC103C00 | 0x25 | 0x26 |
| DMAC_SSI34 | 0xEC10D000 | 0x27 | 0x28 |
| DMAC_SSI35 | 0xEC10D400 | 0x29 | 0x2A |
| DMAC_SSI36 | 0xEC10D800 | 0x2B | 0x2C |
| DMAC_SSI37 | 0xEC10DC00 | 0x2D | 0x2E |
| DMAC_SSI40 | 0xEC104000 | 0x71 | 0x72 |
| DMAC_SSI41 | 0xEC104400 | 0x17 | 0x18 |
| DMAC_SSI42 | 0xEC104800 | 0x19 | 0x1A |
| DMAC_SSI43 | 0xEC104C00 | 0x1B | 0x1C |
| DMAC_SSI44 | 0xEC10E000 | 0x1D | 0x1E |
| DMAC_SSI45 | 0xEC10E400 | 0x1F | 0x20 |
| DMAC_SSI46 | 0xEC10E800 | 0x31 | 0x32 |
| DMAC_SSI47 | 0xEC10EC00 | 0x33 | 0x34 |
| DMAC_SSI5 | 0xEC105000 | 0x73 | 0x74 |
| DMAC_SSI6 | 0xEC106000 | 0x75 | 0x76 |
| DMAC_SSI7 | 0xEC107000 | 0x79 | 0x7A |
| DMAC_SSI8 | 0xEC108000 | 0x7B | 0x7C |
| DMAC_SSI90 | 0xEC109000 | 0x7D | 0x7E |
| DMAC_SSI91 | 0xEC109400 | 0x7F | 0x80 |
| DMAC_SSI92 | 0xEC109800 | 0x81 | 0x82 |
| DMAC_SSI93 | 0xEC109C00 | 0x83 | 0x84 |
| DMAC_SSI94 | 0xEC10F000 | 0xA3 | 0xA4 |
| DMAC_SSI95 | 0xEC10F400 | 0xA5 | 0xA6 |
| DMAC_SSI96 | 0xEC10F800 | 0xA7 | 0xA8 |
| DMAC_SSI97 | 0xEC10FC00 | 0xA9 | 0xAA |
| DMAC_DTCPPP0 | 0xEC120000 | 0xBF | 0xC0 |
| DMAC_DTCPPP1 | 0xEC120400 | 0xD5 | 0xD6 |
| DMAC_DTCPCP0 | 0xEC124000 | 0xD7 | 0xD8 |
| DMAC_DTCPCP1 | 0xEC124400 | 0xD9 | 0xDA |
| DMAC_SCU_SRCI0 | 0xEC000000 | 0x85 | 0xff |
| DMAC_SCU_SRCI1 | 0xEC000400 | 0x87 | 0xff |
| DMAC_SCU_SRCI2 | 0xEC000800 | 0x89 | 0xff |
| DMAC_SCU_SRCI3 | 0xEC000C00 | 0x8B | 0xff |
| DMAC_SCU_SRCI4 | 0xEC001000 | 0x8D | 0xff |
| DMAC_SCU_SRCI5 | 0xEC001400 | 0x8F | 0xff |
| DMAC_SCU_SRCI6 | 0xEC001800 | 0x91 | 0xff |
| DMAC_SCU_SRCI7 | 0xEC001C00 | 0x93 | 0xff |
| DMAC_SCU_SRCI8 | 0xEC002000 | 0x95 | 0xff |
| DMAC_SCU_SRCI9 | 0xEC002400 | 0x97 | 0xff |
| DMAC_SCU_SRCO0 | 0xEC004000 | 0xff | 0x9A |
| DMAC_SCU_SRCO1 | 0xEC004400 | 0xff | 0x9C |
| DMAC_SCU_SRCO2 | 0xEC004800 | 0xff | 0x9E |
| DMAC_SCU_SRCO3 | 0xEC004C00 | 0xff | 0xA0 |
| DMAC_SCU_SRCO4 | 0xEC005000 | 0xff | 0xB0 |
| DMAC_SCU_SRCO5 | 0xEC005400 | 0xff | 0xB2 |
| DMAC_SCU_SRCO6 | 0xEC005800 | 0xff | 0xB4 |
| DMAC_SCU_SRCO7 | 0xEC005C00 | 0xff | 0xB6 |
| DMAC_SCU_SRCO8 | 0xEC006000 | 0xff | 0xB8 |
| DMAC_SCU_SRCO9 | 0xEC006400 | 0xff | 0xBA |
| DMAC_SCU_CMD0 | 0xEC008000 | 0xff | 0xBC |
| DMAC_SCU_CMD1 | 0xEC008400 | 0xff | 0xBE |
| DMAC_MLM0 | 0xEC020000 | 0xDB | 0xDC |
| DMAC_MLM1 | 0xEC020400 | 0xE3 | 0xE4 |
| DMAC_MLM2 | 0xEC020800 | 0xE5 | 0xE6 |
| DMAC_MLM3 | 0xEC020C00 | 0xE7 | 0xE8 |
| DMAC_MLM4 | 0xEC021000 | 0xF3 | 0xF4 |
| DMAC_MLM5 | 0xEC021400 | 0xF5 | 0xF6 |
| DMAC_MLM6 | 0xEC021800 | 0xF7 | 0xF8 |
| DMAC_MLM7 | 0xEC021C00 | 0xF9 | 0xFA |

## Function definition

### xa_dmac_get_register_addr

DD_PLG_TDM_03_001

| Syntax | inline UWORD32 xa_dmac_get_register_addr(UWORD32 reg_addr0, DMAC_CHN n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to get DMAC channel N register’s address. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | UWORD32 | reg_addr0 | I | Pointer to register of channel 0 |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |
| Return value | The value of register channel N |  |  |  |
| Description | - xa_dmac_get_register_addr command processing: - Get DMAC channel N register’s address |  |  |  |

[Covers: FD_PLG_TDM_005, FD_PLG_TDM_017]

Figure 3-1 xa_dmac_get_register_addr flowchart

### xa_dmac_set_stage_number

DD_PLG_TDM_03_002

| Syntax | static VOID xa_dmac_set_stage_number(UWORD32 n, UWORD32 stg_num) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set stage number. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | UWORD32 | n | I | Channel number Valid range: [0: 31] |
|  | UWORD32 | stg_num | I | Stage number Valid values: greater than zero |
| Return value | None |  |  |  |
| Description | - xa_dmac_set_stage_number command processing: - Set stage for DMA channel N |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-2 xa_dmac_set_stage_number flowchart

### xa_dmac_reset_descriptor

DD_PLG_TDM_03_003

| Syntax | static DMAC_ERROR_CODE xa_dmac_reset_descriptor(DMAC_CHN n) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function | This function is to reset descriptor of DMA channel N. |  |  |  |  |
| Arguments | Type | Name | I/O | Description |  |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |  |
| Return value | DMAC_ERROR_NONE |  |  |  | Always success |
| Description | - xa_dmac_reset_descriptor command processing: - Reset descriptor of DMA channel N |  |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-3 xa_dmac_reset_descriptor flowchart

### xa_dmac_start_descriptor

DD_PLG_TDM_03_004

| Syntax | static DMAC_ERROR_CODE xa_dmac_start_descriptor(DMAC_CHN n) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function | This function is to start descriptor of DMA channel N. |  |  |  |  |
| Arguments | Type | Name | I/O | Description |  |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |  |
| Return value | DMAC_ERROR_NONE |  |  |  | Normal end |
| Description | - xa_dmac_start_descriptor command processing: - Start descriptor of DMA channel N |  |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-4 xa_dmac_start_descriptor flowchart

### xa_dmac_enable_ch

DD_PLG_TDM_03_005

| Syntax | static inline DMAC_ERROR_CODE xa_dmac_enable_ch(DMAC_CHN n) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function | This function is to enable DMA channel N. |  |  |  |  |
| Arguments | Type | Name | I/O | Description |  |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |  |
| Return value | DMAC_ERROR_BUSY |  |  |  | Current ADMAC is running |
|  | DMAC_ERROR_NONE |  |  |  | Normal end |
| Description | - xa_dmac_enable_ch command processing: - Enable DMA channel N |  |  |  |  |

[Covers: FD_PLG_TDM_005, FD_PLG_TDM_017]

Figure 3-5 xa_dmac_enable_ch flowchart

### xa_dmac_disable_ch

DD_PLG_TDM_03_006

| Syntax | static inline DMAC_ERROR_CODE xa_dmac_disable_ch(DMAC_CHN n) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function | This function is to disable DMA channel N. |  |  |  |  |
| Arguments | Type | Name | I/O | Description |  |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |  |
| Return value | DMAC_ERROR_OUT_RANGE |  |  |  | DMAC module is invalid |
|  | DMAC_ERROR_NONE |  |  |  | Normal end |
| Description | - xa_dmac_disable_ch command processing: - Disable DMA channel N |  |  |  |  |

[Covers: FD_PLG_TDM_005, FD_PLG_TDM_017]

Figure 3-6 xa_dmac_disable_ch flowchart

### xa_dmac_enable_module

DD_PLG_TDM_03_007

| Syntax | static inline VOID xa_dmac_enable_module(DMAC_CHN n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to enable DMAC module. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |
| Return value | None |  |  |  |
| Description | - xa_dmac_enable_module command processing: - Set ‘1’ to DE bit of register XF_RCAR_DMAOR_L or XF_RCAR_DMAOR_U depending channel number to enable DMAC module |  |  |  |

[Covers: FD_PLG_TDM_005, FD_PLG_TDM_017]

Figure 3-7 xa_dmac_enable_module flowchart

### xa_dmac_clear_channel

DD_PLG_TDM_03_008

| Syntax | static inline void xa_dmac_clear_channel(DMAC_CHN n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to clear value set in DMAC channel N registers. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |
| Return value | None |  |  |  |
| Description | - xa_dmac_clear_channel command processing: - Clear value set in DMAC channel N registers |  |  |  |

[Covers: FD_PLG_TDM_005, FD_PLG_TDM_017]

Figure 3-8 xa_dmac_clear_channel flowchart

### xa_dmac_check_stage

DD_PLG_TDM_03_009

| Syntax | inline WORD32 xa_dmac_check_stage(DMAC_CHN n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to check the current stage of DMAC module. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |
| Return value | The stage of ADMAC module |  |  |  |
| Description | - xa_dmac_check_stage command processing: - Check the current stage of DMAC module |  |  |  |

[Covers: FD_PLG_TDM_017]

Figure 3-9 xa_dmac_check_stage flowchart

### xa_dmac_check_and_clear_interrupt

DD_PLG_TDM_03_010

| Syntax | inline WORD32 admac_check_and_clear_interrupt(DMAC_CHN n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to check and clear interrupt of DMAC channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |
| Return value | The interrupt_flag |  |  |  |
| Description | - xa_dmac_check_and_clear_interrupt command processing: - Check and clear the interrupt of DMAC channel N |  |  |  |

[Covers: FD_PLG_TDM_017]

Figure 3-10 xa_dmac_check_and_clear_interrupt flowchart

### xa_dmac_start

DD_PLG_TDM_03_011

| Syntax | inline DMAC_ERROR_CODE xa_dmac_start(DMAC_CHN n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to start DMAC channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |
| Return value | DMAC_ERROR_BUSY |  | Cannot enable ADMAC transfer on the current ADMAC channel |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - xa_dmac_start command processing: - Start the DMAC channel N |  |  |  |

[Covers: FD_PLG_TDM_005, FD_PLG_TDM_017]

Figure 3-11 xa_dmac_start flowchart

### xa_dmac_stop

DD_PLG_TDM_03_012

| Syntax | DMAC_ERROR_CODE xa_dmac_stop(DMAC_CHN n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to stop DMAC channel N. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |
| Return value | DMAC_ERROR_INVALID |  | Cannot disable ADMAC transfer on the current ADMAC channel |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - xa_dmac_stop command processing: - Stop the DMAC channel N and clear value set on all of its registers |  |  |  |

[Covers: FD_PLG_TDM_017]

Figure 3-12 xa_dmac_stop flowchart

### xa_dmac_descriptor_setup

DD_PLG_TDM_03_013

| Syntax | static VOID xa_dmac_descriptor_setup(DMAC_CHN n, const pVOID descript_mem) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set DMAC channel N’s descriptor. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_CHN | n | I | Channel number Valid range: [0: 31] |
|  | const pVOID | descript_mem | I | Pointer to descriptor memory |
| Return value | None |  |  |  |
| Description | - xa_dmac_descriptor_setup command processing: - Set DMAC channel N’s descriptor |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-13 xa_dmac_descriptor_setup flowchart

### xa_dmac_stage_config

DD_PLG_TDM_03_014

| Syntax | static VOID xa_dmac_stage_config(const pVOID src, const pVOID dst, const pVOID stage_mem, UWORD32 trans_num) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set DMAC stage. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | const pVOID | src | I | Source address |
|  | const pVOID | dst | I | Destination address |
|  | const pVOID | stage_mem | I | Stage memory |
|  | UWORD32 | trans_num | I | Transmission number in a time |
| Return value | None |  |  |  |
| Description | - xa_dmac_stage_config command processing: - Set DMAC stage |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-14 xa_dmac_stage_config flowchart

### xa_dmac_pre_config

DD_PLG_TDM_03_015

| Syntax | static inline DMAC_ERROR_CODE admac_pre_config(DMAC_CHN dma_channel, UWORD32 src_reqst, UWORD32 reqst_flag, const void *descript_mem, UWORD32 stg_num, DMAC_MODE mode, UWORD32 transfer_size) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set DMAC parameters. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_CHN | dma_channel | I | DMAC channel Valid range: [0: 31] |
|  | UWORD32 | src_reqst | I | Source request |
|  | UWORD32 | reqst_flag | I | Source or destination is requested Valid values: 1/2 |
|  | const pVOID | descript_mem | I | Descriptor memory address |
|  | UWORD32 | stg_num | I | Stage number Valid values: greater than zero |
|  | DMAC_MODE | mode | I | Operating mode Valid values: 1/2 |
|  | UWORD32 | transfer_size | I | Transmission unit Valid values: 4 |
| Return value | DMAC_ERROR_INVALID |  | Stage number is invalid Transfer size is invalid |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - xa_dmac_pre_config command processing: - Set DMAC parameters |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-15 xa_dmac_pre_config flowchart

### xa_dmac_post_config

DD_PLG_TDM_03_016

| Syntax | static DMAC_ERROR_CODE xa_dmac_post_config(DMAC_CHN n) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to post set DMAC parameters for DMAC channel N |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_CHN | n | I | Channel number Valid values: [0: 31] |
| Return value | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - xa_dmac_post_config command processing: - Reset the descriptor and then start it |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-16 xa_dmac_post_config flowchart

### xa_dmac_check_valid_channel

DD_PLG_TDM_03_017

| Syntax | DMAC_ERROR_CODE xa_dmac_check_valid_channel(pVOID pch) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to check availability of DMACs and choose an available one for operation. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | pVOID | pch | I/O | Pointer to channel of DMAC Valid values: [0: 31] |
| Return value | DMAC_ERROR_OUT_RANGE |  | DMAC module is invalid |  |
|  | DMAC_ERROR_BUSY |  | No available DMAC module |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - xa_dmac_check_valid_channel command processing: - Check availability of DMACs - Choose an available one for operation |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-17 xa_dmac_check_valid_channel flowchart

### xa_dmac_setting_use_descriptor

DD_PLG_TDM_03_018

| Syntax | DMAC_ERROR_CODE xa_dmac_setting_use_descriptor(const DMAC_PARAMS *dmac_set) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function | This function is to set DMAC module using descriptor memory. |  |  |  |  |
| Arguments | Type | Name | I/O |  | Description |
|  | const DMAC_PARAMS * | dmac_set | I |  | Pointer to DMAC parameter structure |
| Return value | DMAC_ERROR_INVALID |  |  | Stage number is invalid Request address is invalid Transfer size is invalid |  |
|  | DMAC_ERROR_NONE |  |  | Normal end |  |
| Description | - xa_dmac_setting_use_descriptor command processing: - Pre set up DMAC module - Set up all stages based on transmission mode - Post set up DMAC module |  |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-18 xa_dmac_setting_use_descriptor flowchart

### xa_dmac_setting_nouse_descriptor

DD_PLG_TDM_03_019

| Syntax | DMAC_ERROR_CODE xa_dmac_setting_nouse_descriptor(const DMAC_PARAMS *dmac_set) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set DMAC module without using descriptor memory. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | const DMAC_PARAMS * | dmac_set | I | Pointer to DMAC parameter structure |
| Return value | DMAC_ERROR_INVALID |  | Transfer size is invalid Source request address is invalid Cannot disable ADMAC transfer on the current ADMAC channel |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - xa_dmac_setting_nouse_descriptor command processing: - Set registers necessary for ADAMC execution without descriptor memory involvement |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-19 xa_dmac_setting_nouse_descriptor flowchart

### xa_dmac_resetup

DD_PLG_TDM_03_020

| Syntax | inline DMAC_ERROR_CODE admac_resetup(DMAC_PARAMS dmac_set)s |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to re-setup DMAC module. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | DMAC_PARAMS | dmac_set | I | DMAC parameter structure |
| Return value | DMAC_ERROR_INVALID |  | Cannot disable transfer on the current DMAC module Cannot enable transfer on the current DMAC module |  |
|  | DMAC_ERROR_NONE |  | Normal end |  |
| Description | - xa_dmac_resetup command processing: - Re-setup DMAC module |  |  |  |

[Covers: FD_PLG_TDM_017]

Figure 3-20 xa_dmac_resetup flowchart

# Revision history

| Version | Date | Page | Content | Approved | Changed |
| --- | --- | --- | --- | --- | --- |
| 1.0.0 | Nov 14 2018 | - | First Edition issued | Vu Phan | Nguyen Dang |
| 1.1.0 | Dec 10 2018 | - | Add traceability ID | Vu Phan | Nguyen Dang |
| 1.2.0 | Jan 03, 2019 | - | Add range for input parameters | Vu Phan | Tien Tran |