Document Type: Detail Design

Document Name:

ADSP FRAMEWORK: FIFO DRIVER

Renesas Electronics Corporation

Renesas Design Vietnam Co. Ltd

R-Car Software Solution 2 Group

Middleware 1 Team

Table of Contents

1.	Overview	3

2.	Function list	4

3.	Detail information	5

3.1	Data type definition	5

3.1.1	FIFO_MODULE	5

3.1.2	FIFO_ERROR_CODE	5

3.1.3	fifo_type	5

3.1.4	FIFO_PARAM	6

3.2	Macro definition	7

3.3	Register definition	8

3.4	Function definition	10

3.4.1	xa_fifo_enable	10

3.4.2	xa_fifo_config	12

3.4.3	xa_fifo_get_int_num	15

3.4.4	xa_fifo_register_interrupt_handler	16

3.4.5	xa_fifo_unregister_interrupt_handler	17

3.4.6	xa_fifo_disable_interrupt	18

3.4.7	xa_fifo_enable_interrupt	19

3.4.8	xa_fifo_clear_interrupt	20

3.4.9	xa_fifo_stop	21

3.4.10	xa_fifo_check_available	22

4.	Revision history	24

List of Figures

Figure 1-1 The software architecture	3

Figure 3-1 xa_fifo_enable flowchart	11

Figure 3-3 xa_fifo_config flowchart	14

Figure 3-4 xa_fifo_get_int_num flowchart	15

Figure 3-5 xa_fifo_register_interrupt_handler flowchart	16

Figure 3-6 xa_fifo_unregister_interrupt_handler flowchart	17

Figure 3-7 xa_fifo_disable_interrupt flowchart	18

Figure 3-8 xa_fifo_enable_interrupt flowchart	19

Figure 3-9 xa_fifo_clear_interrupt	20

Figure 3-10 xa_fifo_stop flowchart	21

Figure 3-11 xa_fifo_check_available	23

List of Table

Table 2-1 Function list	4

Table 3-1 FIFO_MODULE type information	5

Table 3-2 FIFO_ERROR_CODE type information	5

Table 3-3 fifo_type type information	5

Table 3-4 FIFO_PARAM type information	6

Table 3-5 Setting registers OF_EN and IF_EN to enable FIFO	10

Table 2-1 Function list	4

Table 3-1 FIFO_MODULE type information	5

Table 3-2 FIFO_ERROR_CODE type information	5

Table 3-3 fifo_type type information	5

Table 3-4 FIFO_PARAM type information	6

Table 3-5 Setting registers OF_EN and IF_EN to enable FIFO	10

# Overview

This section provides an overview of software architecture.

User Application

ADSP Interface

Kernel Space

User Space

ADSP Driver

ARM

Audio HW

ADSP Framework

TDM class

Renderer class

Equalizer class

Capture class

TDM Plugin

ADSP

Plugin

Equalizer Plugin*

Capture Plugin

Renderer Plugin

This document’s target is in side of red square

* not connect to hardware modules

- DAC/

- ADC

FIFO

SCU

PDMA

SSI

SSIU

ADMA

Figure 1-1 The software architecture

# Function list

The following is list of functions:

Table 2-1 Function list

| Type | Function Name | Outline |
| --- | --- | --- |
| API | xa_fifo_enable | This API is to enable the input/output of FIFO |
|  | xa_fifo_config | This API is to set up registers necessary for FIFO module execution |
|  | xa_fifo_register_interrupt_handler | This API is to register interrupt handler for FIFO |
|  | xa_fifo_unregister_interrupt_handler | This API is to unregister interrupt handler for FIFO |
|  | xa_fifo_disable_interrupt | This API is to disable FIFO interrupt |
|  | xa_fifo_enable_interrupt | This API is to enable FIFO interrupt |
|  | xa_fifo_clear_interrupt | This API is to clear FIFO interrupt |
|  | xa_fifo_stop | This API is to stop FIFO operation |
|  | xa_fifo_check_available | This API is to check availability of FIFO module and choose an available one |
| Internal function | xa_fifo_get_int_num | This function is to get the interrupt number |

# Detail information

This section describes detail information of data types, macro definitions, implemented APIs and internal function units, global variable.

## Data type definition

### FIFO_MODULE

The data type FIFO_MODULE is a type-defined enumeration that lists all supported FIFO modules.

Table 3-1 FIFO_MODULE type information

| Member name | Value | Outline |
| --- | --- | --- |
| FIFO0 | 0 | 1st FIFO |
| FIFO1 | 1 | 2nd FIFO |
| FIFO2 | 2 | 3rd FIFO |
| FIFO3 | 3 | 4th FIFO |
| FIFOMAX | 4 | Number of FIFO modules |

### FIFO_ERROR_CODE

The data type FIFO_ERROR_CODE is a type-defined enumeration that lists all error codes for FIFO.

Table 3-2 FIFO_ERROR_CODE type information

| Member name | Value |
| --- | --- |
| FIFO_ERROR_NONE | 0 |
| FIFO_ERROR_INVALID | -1 |
| FIFO_ERROR_INTERNAL | -2 |
| FIFO_ERROR_BUSY | -3 |
| FIFO_ERROR_OUT_RANGE | -4 |

### fifo_type

The data type fifo_type is a type-defined enumeration of indicators of whether FIFO’s input or output is used.

Table 3-3 fifo_type type information

| Member name | Value | Outline |
| --- | --- | --- |
| FIFO_INPUT | 0 | FIFO input is used |
| FIFO_OUTPUT | 1 | FIFO output is used |

### FIFO_PARAM

The data type FIFO_PARAM is a type-defined structure that possesses necessary parameters for FIFO module.

Table 3-4 FIFO_PARAM type information

| Member name | Data type | Outline |
| --- | --- | --- |
| channel | UWORD32 | channel of data used for FIFO module |
| pcm_width | UWORD32 | PCM width of data used for FIFO module |
| sample_rate | UWORD32 | Sampling rate of data used for FIFO module |
| fifo_num | FIFO_MODULE | FIFO module |
| type | fifo_type | FIFO type |
| dev_connect | UWORD32 | Indicator to show which module FIFO connects |
| ssi_idx | UWORD32 | SSI module |

## Macro definition

| Macros | Value | Outline |
| --- | --- | --- |
| XA_FIFO_FULL | 1024 | FIFO full size |
| XA_FIFO_NEARFULL | 768 | FIFO nearly full size |
| XA_FIFO_NEAREMPTY | 64 | FIFO nearly empty size |
| XA_FIFO_EMPTY | 0 | FIFO empty size |
| XA_FIFO_CONNECT_SRC | 0 | FIFO is connected to SRC |
| XA_FIFO_CONNECT_SSI | 1 | FIFO is connected to SSI |

## Register definition

Below is the table listing all items that are macros representing register addresses located in repository s492d/include/sys/xt-shmem/board-rcar/xf-registers.h

| Register | Outline |
| --- | --- |
| XF_RCAR_REG_OF_EN | Register OF_EN |
| XF_RCAR_REG_IF_EN | Register IF_EN |
| XF_RCAR_REG_ADSP_TIMSEL | ADSP Output Timing Select Register |
| XF_RCAR_REG_ADSP_TIMSEL1 | ADSP Output Timing Select Register 1 |
| XF_RCAR_REG_ADSP_TIMSEL2 | ADSP Output Timing Select Register 2 |
| XF_RCAR_REG_ADSP_TIMSEL3 | ADSP Output Timing Select Register 3 |
| XF_RCAR_REG_OF_MSTP | Output FIFO Module Stop Register |
| XF_RCAR_REG_OF_PTRRST | Output FIFO Pointer Reset Register |
| XF_RCAR_REG_OF0_CH | Output FIFO0 Channel Register |
| XF_RCAR_REG_OF1_CH | Output FIFO1 Channel Register |
| XF_RCAR_REG_OF2_CH | Output FIFO2 Channel Register |
| XF_RCAR_REG_OF3_CH | Output FIFO3 Channel Register |
| XF_RCAR_REG_OF_ADINR | Output FIFO0 Audio Information Register |
| XF_RCAR_REG_OF1_ADINR | Output FIFO1 Audio Information Register |
| XF_RCAR_REG_OF2_ADINR | Output FIFO2 Audio Information Register |
| XF_RCAR_REG_OF3_ADINR | Output FIFO3 Audio Information Register |
| XF_RCAR_REG_OF0_INT_TH | Output FIFO0 Interrupt Threshold Register |
| XF_RCAR_REG_OF1_INT_TH | Output FIFO1 Interrupt Threshold Register |
| XF_RCAR_REG_OF2_INT_TH | Output FIFO2 Interrupt Threshold Register |
| XF_RCAR_REG_OF3_INT_TH | Output FIFO3 Interrupt Threshold Register |
| XF_RCAR_REG_OF_INT_CLR | Output FIFO Interrupt Clear Register |
| XF_RCAR_REG_IF_MSTP | Input FIFO Module Stop Register |
| XF_RCAR_REG_IF_PTRRST | Input FIFO Pointer Reset Register |
| XF_RCAR_REG_IF0_CH | Input FIFO0 Channel Register |
| XF_RCAR_REG_IF1_CH | Input FIFO1 Channel Register |
| XF_RCAR_REG_IF2_CH | Input FIFO2 Channel Register |
| XF_RCAR_REG_IF3_CH | Input FIFO3 Channel Register |
| XF_RCAR_REG_IF_ADINR | Input FIFO0 Audio Information Register |
| XF_RCAR_REG_IF1_ADINR | Input FIFO1 Audio Information Register |
| XF_RCAR_REG_IF2_ADINR | Input FIFO2 Audio Information Register |
| XF_RCAR_REG_IF3_ADINR | Input FIFO3 Audio Information Register |
| XF_RCAR_REG_IF0_INT_TH | Input FIFO0 Interrupt Threshold Register |
| XF_RCAR_REG_IF1_INT_TH | Input FIFO1 Interrupt Threshold Register |
| XF_RCAR_REG_IF2_INT_TH | Input FIFO2 Interrupt Threshold Register |
| XF_RCAR_REG_IF3_INT_TH | Input FIFO3 Interrupt Threshold Register |
| XF_RCAR_REG_IF_INT_CLR | Input FIFO Interrupt Clear Register |

## Function definition

### xa_fifo_enable

DD_PLG_TDM_04_001

| Syntax | inline FIFO_ERROR_CODE xa_fifo_enable(FIFO_MODULE fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to enable the input/output of FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_MODULE | fifo_num | I | FIFO module Valid values: [0: 3] |
|  | fifo_type | type | I | FIFO uses its input or output Valid values: 0/1 |
| Return value | FIFO_ERROR_BUSY |  | FIFO module is busy |  |
|  | FIFO_ERROR_NONE |  | Normally end |  |
| Description | - xa_fifo_enable command processing: - Set either register OF_EN or IF_EN based on FIFO type |  |  |  |

[Covers: FD_PLG_TDM_005]

Table 3-5 Setting registers OF_EN and IF_EN to enable FIFO

| fifo_num | type | Register | Bit | Value |
| --- | --- | --- | --- | --- |
| FIFO0 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 0 | 1 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 0 | 1 |
| FIFO1 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 1 | 1 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 1 | 1 |
| FIFO2 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 2 | 1 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 2 | 1 |
| FIFO3 | FIFO_OUTPUT | XF_RCAR_REG_OF_EN | 3 | 1 |
|  | FIFO_INPUT | XF_RCAR_REG_IF_EN | 3 | 1 |

Figure 3-1 xa_fifo_enable flowchart

### xa_fifo_config

DD_PLG_TDM_04_002

| Syntax | VOID xa_fifo_config(FIFO_PARAM fifo_param) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to set up registers necessary for FIFO module execution. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_PARAM | fifo_param | I | Struct of parameters of FIFO |
| Return value | None |  |  |  |
| Description | - xa_fifo_config command processing: - Set registers SSI_BUSIF_MODE, SSI_BUSIF_ADINR, SSI mode register |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-2 xa_fifo_config flowchart

### xa_fifo_get_int_num

DD_PLG_TDM_04_003

| Syntax | static inline WORD32 xa_fifo_get_int_num(FIFO_MODULE fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to get interrupt number of FIFO module. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_MODULE | fifo_num | I | FIFO module Valid values: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: 0/1 |
| Return value | Interrupt number |  |  |  |
| Description | - xa_fifo_get_int_num command processing: - Get interrupt number based on FIFO module and its type (FIFO_INPUT or FIFO_OUTPUT) |  |  |  |

[Covers: FD_PLG_TDM_005, FD_PLG_TDM_017]

Figure 3-3 xa_fifo_get_int_num flowchart

### xa_fifo_register_interrupt_handler

DD_PLG_TDM_04_004

| Syntax | VOID xa_fifo_register_interrupt_handler(FIFO_MODULE fifo_num, fifo_type type, pVOID function_addr, pVOID arg) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to register interrupt handler for FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_MODULE | fifo_num | I | FIFO module Valid values: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: 0/1 |
|  | pVOID | function_addr | I | Pointer to handler function |
|  | pVOID | arg | I | Argument passed to handler |
| Return value | None |  |  |  |
| Description | - xa_fifo_register_interrupt_handler command processing: - Register interrupt handler for FIFO |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-4 xa_fifo_register_interrupt_handler flowchart

### xa_fifo_unregister_interrupt_handler

DD_PLG_TDM_04_005

| Syntax | VOID xa_fifo_unregister_interrupt_handler(FIFO_MODULE fifo_num,fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to unregister interrupt handler for FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_MODULE | fifo_num | I | FIFO module Valid values: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: 0/1 |
| Return value | None |  |  |  |
| Description | - xa_fifo_unregister_interrupt_handler command processing: - Unregister interrupt handler for FIFO |  |  |  |

[Covers: FD_PLG_TDM_017]

Figure 3-5 xa_fifo_unregister_interrupt_handler flowchart

### xa_fifo_disable_interrupt

DD_PLG_TDM_04_006

| Syntax | inline VOID xa_fifo_disable_interrupt(FIFO_MODULE fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to disable interrupt for FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_MODULE | fifo_num | I | FIFO module Valid values: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: 0/1 |
| Return value | None |  |  |  |
| Description | - xa_fifo_disable_interrupt command processing: - Disable FIFO interrupt |  |  |  |

[Covers: FD_PLG_TDM_017]

Figure 3-6 xa_fifo_disable_interrupt flowchart

### xa_fifo_enable_interrupt

DD_PLG_TDM_04_007

| Syntax | inline VOID xa_fifo_enable_interrupt(FIFO_MODULE fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to enable interrupt for FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_MODULE | fifo_num | I | FIFO module Valid values: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: 0/1 |
| Return value | None |  |  |  |
| Description | - xa_fifo_enable_interrupt command processing: - Enable FIFO interrupt |  |  |  |

[Covers: FD_PLG_TDM_005, FD_PLG_TDM_017]

Figure 3-7 xa_fifo_enable_interrupt flowchart

### xa_fifo_clear_interrupt

DD_PLG_TDM_04_008

| Syntax | inline VOID xa_fifo_clear_interrupt(FIFO_MODULE fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to clear FIFO interrupt. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_MODULE | fifo_num | I | FIFO module Valid values: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: 0/1 |
| Return value | None |  |  |  |
| Description | - xa_fifo_clear_interrupt command processing: - Clear FIFO interrupt |  |  |  |

[Covers: FD_PLG_TDM_005, FD_PLG_TDM_017]

Figure 3-8 xa_fifo_clear_interrupt

### xa_fifo_stop

DD_PLG_TDM_04_009

| Syntax | FIFO_ERROR_CODE xa_fifo_stop(FIFO_MODULE fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to stop FIFO. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | FIFO_MODULE | fifo_num | I | FIFO module Valid values: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: 0/1 |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_NONE |  | Normally end |  |
| Description | - xa_fifo_stop command processing: - Stop FIFO operation |  |  |  |

[Covers: FD_PLG_TDM_017]

Figure 3-9 xa_fifo_stop flowchart

### xa_fifo_check_available

DD_PLG_TDM_04_010

| Syntax | inline FIFO_ERROR_CODE xa_fifo_check_available(UWORD32 *fifo_num, fifo_type type) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This function is to check availability of FIFOs and choose an available one for operation. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | UWORD32 * | fifo_num | I/O | Pointer to FIFO number Valid values: [0: 3] |
|  | fifo_type | type | I | FIFO type Valid values: 0/1 |
| Return value | FIFO_ERROR_OUT_RANGE |  | FIFO module is invalid |  |
|  | FIFO_ERROR_BUSY |  | No available FIFO module |  |
|  | FIFO_ERROR_NONE |  | Normally end |  |
| Description | - xa_fifo_check_available command processing: - Check availability of FIFOs - Choose an available one for operation |  |  |  |

[Covers: FD_PLG_TDM_005]

Figure 3-10 xa_fifo_check_available

# Revision history

| Version | Date | Page | Content | Approved | Changed |
| --- | --- | --- | --- | --- | --- |
| 1.0.0 | Nov 14 2018 | - | First Edition issued | Vu Phan | Nguyen Dang |
| 1.1.0 | Dec 10 2018 | - | Add traceability ID | Vu Phan | Nguyen Dang |
| 1.2.0 | Jan 03, 2019 | - | Add range for input parameters | Vu Phan | Tien Tran |