Document Type: Detail Design

Document Name:

ADSP OMX TDM

Renesas Design Vietnam Co. Ltd

R-Car Software Solution 2 Group

Middleware 1 Team

Renesas Electronics Corporation

Table of Contents

1.	Overview	3

2.	Function list	4

3.	Detail information	5

3.1	Data type and Macro definition	5

3.2	Function definition	6

3.2.1	OMX TDM_Capture	6

3.2.1.1	TDM_CAPTURE_Setup	6

3.2.1.2	TDM_CAPTURE_RuntimeInit	7

3.2.1.3	TDM_CAPTURE_GetParam	8

3.2.1.4	TDM_CAPTURE_TimeStamp	9

3.2.1.5	TDM_CAPTURE_GetParameter	10

3.2.1.6	TDM_CAPTURE_SetParameter	11

3.2.1.7	TDM_CAPTURE_ComponentInit	12

3.2.1.8	TDM_CAPTURE_ComponentDeInit	13

3.2.1.9	TDM_CAPTURE_ComponentCreate	14

3.2.2	OMX TDM RENDERER	16

3.2.2.1	TDM_RENDERER_Setup	16

3.2.2.2	RENDERER_RuntimeInit	17

3.2.2.3	TDM_RENDERER_GetParam	18

3.2.2.4	TDM_RENDERER_TimeStamp	19

3.2.2.5	TDM_RENDERER_GetParameter	20

3.2.2.6	TDM_RENDERER_SetParameter	21

3.2.2.7	TDM_RENDERER_ComponentInit	22

3.2.2.8	TDM_RENDERER_ComponentDeInit	23

3.2.2.9	TDM_RENDERER_ComponentCreate	24

4.	Revision history	26

List of Figures

Figure 1-1 The software architecture	3

Figure 3-1 TDM_CAPTURE_Setup flowchart	6

Figure 3-2 TDM_CAPTURE_RuntimeInit flowchart	7

Figure 3-3 TDM_CAPTURE_GetParam flowchart	8

Figure 3-4 TDM_CAPTURE_TimeStamp flowchart	9

Figure 3-5 TDM_CAPTURE_GetParameter flowchart	10

Figure 3-6 TDM_CAPTURE_SetParameter flowchart	11

Figure 3-7 TDM_CAPTURE_ComponentInit flowchart	12

Figure 3-8 TDM_CAPTURE_ComponentDeInit flowchart	13

Figure 3-9 TDM_CAPTURE_ComponentCreate flowchart	15

Figure 3-12 TDM_RENDERER_Setup flowchart	16

Figure 3-13 TDM_RENDERER_GetParam flowchart	18

Figure 3-14 TDM_RENDERER_TimeStamp flowchart	19

Figure 3-15 TDM_RENDERER_GetParameter flowchart	20

Figure 3-16 TDM_RENDERER_SetParameter flowchart	21

Figure 3-17 TDM_RENDERER_ComponentInit flowchart	22

Figure 3-18 TDM_RENDERER_ComponentDeInit flowchart	23

Figure 3-19 TDM_RENDERER_ComponentCreate flowchart	25

List of Table

Table 2-1 Function list of OMX TDM Capture	4

Table 2-2 Function list of OMX TDM Renderer	4

Table 3-1 XAOMX_TDM_CAP type structure information	5

Table 3-2 XAOMX_TDM_RDR type structure information	5

Table 3-3 Macro definitions of OMX TDM Capture	5

Table 3-4 Macro definitions of OMX TDM Renderer	5

Table 2-1 Function list of OMX TDM Capture	4

Table 2-2 Function list of OMX TDM Renderer	4

Table 3-1 XAOMX_TDM_CAP type structure information	5

Table 3-2 XAOMX_TDM_RDR type structure information	5

Table 3-3 Macro definitions of OMX TDM Capture	5

Table 3-4 Macro definitions of OMX TDM Renderer	5

# Overview

In this chapter, overview of OMX TDM interface is explained.

User Application

OMX TDM

OMX Renderer/Capture

OMX Equalizer

User Space

ADSP Interface for Linux (base)

Kernel Space

ARM

ADSP Driver for Linux

Audio HW

ADSP

NC/EC

ADSP Framework

Renderer/Capture Plugin

TDM Plugin

Equalizer Plugin

Codec

This document’s target is in side of red square.

DAC/ADC

SRC/CTU/MIX/ DVC/ SSI/

ADMA

Figure 1-1 The software architecture

# Function list

The following is list of functions:

Table 2-1 Function list of OMX TDM Capture

| No. | Function Name | Outline |
| --- | --- | --- |
| 1 | TDM_CAPTURE_Setup | TDM Capture prepare codec setup parameters. |
| 2 | TDM_CAPTURE_RuntimeInit | Get DMA channel 1 value to initialize output port of the component. |
| 3 | TDM_CAPTURE_GetParam | Get DMA channel 1 value from response structure. |
| 4 | TDM_CAPTURE_TimeStamp | Calculate timestamp for current output buffer and next buffer. |
| 5 | TDM_CAPTURE_GetParameter | Get TDM Capture parameter. |
| 6 | TDM_CAPTURE_SetParameter | Set TDM Capture parameter. |
| 7 | TDM_CAPTURE_ComponentDeInit | Destroy Capture component and private data. |
| 8 | TDM_CAPTURE_ComponentInit | Initialize base codec interface, parameters, PCM and TDM Capture format defaults. |
| 9 | TDM_CAPTURE_ComponentCreate | Create base component and initialize the TDM Capture component. |

Table 2-2 Function list of OMX TDM Renderer

| No. | Function Name | Outline |
| --- | --- | --- |
| 1 | TDM_RENDERER_Setup | TDM Renderer prepare codec setup parameters. |
| 2 | TDM_RENDERER_RuntimeInit | This is a dummy function, not use in TDM Renderer runtime initialization. |
| 3 | TDM_RENDERER_GetParam | This is to check the message length is correct and return 0. |
| 4 | TDM_RENDERER_TimeStamp | Calculate timestamp for current input buffer and next buffer. |
| 5 | TDM_RENDERER_GetParameter | Get TDM Renderer parameter. |
| 6 | TDM_RENDERER_SetParameter | Set TDM Renderer parameter. |
| 7 | TDM_RENDERER_ComponentDeInit | Destroy TDM Renderer component and private data. |
| 8 | TDM_RENDERER_ComponentInit | Initialize base codec interface, parameters, PCM and TDM Renderer format. |
| 9 | TDM_RENDERER_ComponentCreate | Create base component and initialize the TDM Renderer component. |

# Detail information

This section describes detail information of data types, macro definitions.

## Data type and Macro definition

Table 3-1 XAOMX_TDM_CAP type structure information

| Member name | Outline |
| --- | --- |
| XAOMXCodecBase base | Generic codec structure. |
| OMX_AUDIO_PARAM_PCMMODETYPE sPCM | PCM-specific parameters (output port). |
| XAOMX_AUDIO_PARAM_TDM_CAPTURE sTDM_CAPTURE | TDM Capture parameters. |

Note:

Structure XAOMXCodecBase is a base codec structure of OMX in omx-codec-base.h

Structure OMX_AUDIO_PARAM_PCMMODETYPE is a PCM format description in OMX_Audio.h

Structure XAOMX_AUDIO_PARAM_TDM_CAPTURE is a parametric TDM Capture settings structure.

Table 3-2 XAOMX_TDM_RDR type structure information

| Member name | Outline |
| --- | --- |
| XAOMXCodecBase base | Generic codec structure. |
| OMX_AUDIO_PARAM_PCMMODETYPE sPCM | PCM-specific parameters (input port). |
| XAOMX_AUDIO_PARAM_TDM_RENDERER sTDM_RENDERER | TDM Renderer parameters. |

Note:

Structure XAOMX_AUDIO_PARAM_TDM_RENDERER is a parametric TDM Renderer settings structure.

Table 3-3 Macro definitions of OMX TDM Capture

| Macro | Value | Outline |
| --- | --- | --- |
| NUM_OUTPUT_BUFFERS | 4 | Total amount of output buffers. |
| OUTPUT_BUFFER_LENGTH | 4096 | Default output buffer length. |
| BUFFER_ALIGNMENT | 32 | Required data alignment of TDM Capture. |

Table 3-4 Macro definitions of OMX TDM Renderer

| Macro | Value | Outline |
| --- | --- | --- |
| NUM_INPUT_BUFFERS | 4 | Total amount of input buffers. |
| INPUT_BUFFER_LENGTH | 4096 | Default input buffer length. |
| BUFFER_ALIGNMENT | 32 | Required data alignment of TDM Renderer. |

## Function definition

### OMX TDM_Capture

#### TDM_CAPTURE_Setup

DD_API_TDM_01_001

| Syntax | static s32 TDM_CAPTURE_Setup(XAOMXCodecBase *pBase, xf_set_param_msg_t *msg) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | TDM Capture prepare codec setup parameters. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I | Pointer to base codec structure of TDM Capture component. |
|  | xf_set_param_msg_t* | msg | O | Message send to TDM Capture plugin. |
| Return value | length |  | Number of parameters want to set for TDM Capture plugin. |  |
| Description | - TDM_CAPTURE_Setup processing: - Prepare PCM width, channels, sample rate, input1, 2; DMA channel 1, 2; out sample rate, volume rate values into command message. |  |  |  |

[Covers: FD_API_TDM_003]

Figure 3-1 TDM_CAPTURE_Setup flowchart

#### TDM_CAPTURE_RuntimeInit

DD_API_TDM_01_002

| Syntax | static s32 TDM_CAPTURE_RuntimeInit(XAOMXCodecBase *pBase, xf_start_msg_t *msg) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Get DMA channel 1 value to initialize output port of the component. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I | Pointer to base codec structure of TDM Capture component. |
|  | xf_start_msg_t* | msg | O | Message send to TDM Capture plugin. |
| Return value | length |  | Number of parameters want to set for TDM Capture plugin. |  |
| Description | - TDM_CAPTURE_RuntimeInit processing: - Save DMA channel 1 into command structure. |  |  |  |

[Covers: FD_API_TDM_003]

Figure 3-2 TDM_CAPTURE_RuntimeInit flowchart

#### TDM_CAPTURE_GetParam

DD_API_TDM_01_003

| Syntax | static s32 TDM_CAPTURE_GetParam(XAOMXCodecBase *pBase, xf_get_param_msg_t *msg, u32 length) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Get DMA channel 1 value from response structure. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I/O | Pointer to base codec structure of TDM Capture component. |
|  | xf_get_param_msg_t * | msg | I | Message receive from TDM Capture plugin. |
|  | u32 | length | I | Length of message. |
| Return value | 0 |  | Normal return. |  |
|  | -EBADF |  | The message length is invalid |  |
| Description | - TDM_CAPTURE_GetParam processing: - Get DMA channel 1 values from response structure. |  |  |  |

[Covers: FD_API_TDM_003]

Figure 3-3 TDM_CAPTURE_GetParam flowchart

#### TDM_CAPTURE_TimeStamp

DD_API_TDM_01_004

| Syntax | static void TDM_CAPTURE_TimeStamp(XAOMXCodecBase *pBase, OMX_BUFFERHEADERTYPE *pBufHdr) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Calculate timestamp for current output buffer and next buffer. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I/O | Pointer to base codec structure of TDM Capture component. |
|  | OMX_BUFFERHEADERTYPE* | pBufHdr | O | Output buffer. |
| Return value | None. |  |  |  |
| Description | - TDM_CAPTURE_TimeStamp processing: - Add current timestamp to the output buffer. - Calculate timestamp for the next buffer. |  |  |  |

[Covers: FD_API_TDM_003]

Figure 3-4 TDM_CAPTURE_TimeStamp flowchart

#### TDM_CAPTURE_GetParameter

DD_API_TDM_01_005

| Syntax | static OMX_ERRORTYPE TDM_CAPTURE_GetParameter(XAOMXCodecBase *pBase, OMX_INDEXTYPE nIndex, OMX_PTR pParam) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Get parameter and save to pParam. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I/O | Pointer to base codec structure of TDM Capture component. |
|  | OMX_INDEXTYPE | nIndex | I | Index of parameter structure. Valid values: OMX_IndexParamAudioPcm XAOMX_IndexParamAudioTDMCapture OMX_IndexParamCompBufferSupplier |
|  | OMX_PTR | pParam | I | Pointer to parameter structure want to save |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorUnsupportedIndex |  | Wrong Index of structure |  |
| Description | - TDM_CAPTURE_GetParameter processing: - Get parameters of OMX_AUDIO_PARAM_PCMMODETYPE, XAOMX_AUDIO_PARAM_TDM_CAPTURE, OMX_PARAM_BUFFERSUPPLIERTYPE structure base on nIndex. |  |  |  |

[Covers: FD_API_TDM_004]

Figure 3-5 TDM_CAPTURE_GetParameter flowchart

#### TDM_CAPTURE_SetParameter

DD_API_TDM_01_006

| Syntax | static OMX_ERRORTYPE TDM_CAPTURE_SetParameter(XAOMXCodecBase *pBase, OMX_INDEXTYPE nIndex, OMX_PTR pParam) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Set parameter from pParam and save to parameter structure. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I/O | Pointer to base codec structure of TDM Capture component. |
|  | OMX_INDEXTYPE | nIndex | I | Index of parameter structure. Valid values: OMX_IndexParamAudioPcm XAOMX_IndexParamAudioTDMCapture OMX_IndexParamCompBufferSupplier |
|  | OMX_PTR | pParam | O | Pointer to parameter structure want to set. |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorUnsupportedIndex |  | Wrong Index of structure. |  |
|  | OMX_ErrorBadPortIndex |  | Port index is invalid. |  |
| Description | - TDM_CAPTURE_SetParameter processing: - Set parameters of OMX_AUDIO_PARAM_PCMMODETYPE, XAOMX_AUDIO_PARAM_TDM_CAPTURE, OMX_PARAM_BUFFERSUPPLIERTYPE structure base on nIndex. |  |  |  |

[Covers: FD_API_TDM_005]

Figure 3-6 TDM_CAPTURE_SetParameter flowchart

#### TDM_CAPTURE_ComponentInit

DD_API_TDM_01_007

| Syntax | static OMX_ERRORTYPE CAPTURE_ComponentInit(xf_proxy_t *proxy, OMX_HANDLETYPE hComponent, OMX_PTR pAppData, OMX_CALLBACKTYPE *pCallbacks) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Initialize base codec interface, parameters, PCM and TDM Capture format. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | xf_proxy_t * | proxy | O | OpenMAX interface proxy. |
|  | OMX_HANDLETYPE | hComponent | O | Pointer to TDM Capture component handle. |
|  | OMX_PTR | pAppData | I | Pointer to pAppData of TDM Capture component. |
|  | OMX_CALLBACKTYPE | pCallbacks | I | Pointer to Callbacks of TDM Capture component. |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorInsufficientResources |  | Initialize base codec interface is fail. |  |
| Description | - TDM_CAPTURE_ComponentInit processing: - Initialize base codec interface, parameters, PCM and TDM Capture format. |  |  |  |

[Covers: FD_API_TDM_001]

Figure 3-7 TDM_CAPTURE_ComponentInit flowchart

#### TDM_CAPTURE_ComponentDeInit

DD_API_TDM_01_008

| Syntax | static OMX_ERRORTYPE TDM_CAPTURE_ComponentDeInit(OMX_HANDLETYPE hComponent) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Destroy TDM Capture component and private data. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | OMX_HANDLETYPE | hComponent | O | Pointer to TDM Capture component handle. |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorBadParameter |  | Private data is NULL. |  |
| Description | - TDM_CAPTURE_ComponentDeInit processing: - Destroy TDM Capture component and private data. |  |  |  |

[Covers: FD_API_TDM_002]

Figure 3-8 TDM_CAPTURE_ComponentDeInit flowchart

#### TDM_CAPTURE_ComponentCreate

DD_API_TDM_01_009

| Syntax | OMX_ERRORTYPE TDM_CAPTURE_ComponentCreate(xf_proxy_t *proxy, OMX_HANDLETYPE *hComponent, OMX_PTR pAppData, OMX_CALLBACKTYPE *pCallbacks) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Create base component and initialize the TDM Capture component. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | xf_proxy_t* | proxy | O | OpenMAX interface proxy. |
|  | OMX_HANDLETYPE* | hComponent | O | Pointer to TDM Capture component handle. |
|  | OMX_PTR | pAppData | I | Pointer to pAppData of TDM Capture component. |
|  | OMX_CALLBACKTYPE * | pCallbacks | I | Pointer to Callbacks of TDM Capture component. |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorInsufficientResources |  | Cannot create the base component. Cannot allocate private memory. Initialize the TDM Capture component is fail. |  |
| Description | - TDM_CAPTURE_ComponentCreate processing: - Create the base component. - Allocate the private memory. - Initialize the TDM Capture component. |  |  |  |

[Covers: FD_API_TDM_001]

Figure 3-9 TDM_CAPTURE_ComponentCreate flowchart

### OMX TDM RENDERER

#### TDM_RENDERER_Setup

DD_API_TDM_01_010

| Syntax | static s32 TDM_RENDERER_Setup(XAOMXCodecBase *pBase, xf_set_param_msg_t *msg) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | TDM Renderer prepare codec setup parameters. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I | Pointer to base codec structure of TDM Renderer component. |
|  | xf_set_param_msg_t* | msg | O | Message send to TDM Renderer plugin. |
| Return value | length |  | Number of parameters want to set for TDM Renderer plugin. |  |
| Description | - TDM_Renderer_Setup processing: - Prepare PCM width, channels, sample rate, output1, 2; DMA channel 1, 2; out sample rate, volume rate values into command message. |  |  |  |

[Covers: FD_API_TDM_003]

Figure 3-10 TDM_RENDERER_Setup flowchart

#### RENDERER_RuntimeInit

DD_API_TDM_01_011

| Syntax | static s32 TDM_RENDERER_RuntimeInit(XAOMXCodecBase*pBase, xf_start_msg_t *msg) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This is a dummy function, not use in TDM Renderer runtime initialization. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I | Pointer to base codec structure of TDM Renderer component. |
|  | xf_start_msg_t* | msg | O | Message send to TDM Renderer plugin. |
| Return value | 0 |  | No parameter need to set to plugin |  |
| Description | - TDM_RENDERER_RuntimeInit processing: - Return 0. |  |  |  |

[Covers: FD_API_TDM_003]

#### TDM_RENDERER_GetParam

DD_API_TDM_01_012

| Syntax | static s32 TDM_RENDERER_GetParam(XAOMXCodecBase *pBase, xf_get_param_msg_t *msg, u32 length) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | This is to check the message length is correct and return 0. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I | Pointer to base codec structure of TDM RENDERER component. |
|  | xf_get_param_msg_t * | msg | I | Message receive from TDM RENDERER plugin. |
|  | u32 | length | I | Length of message. |
| Return value | 0 |  | Normal return |  |
|  | -EBADF |  | The message length is invalid |  |
| Description | - TDM_RENDERER_GetParam processing: - Check the message length is correct and return 0. |  |  |  |

[Covers: FD_API_TDM_003]

Figure 3-11 TDM_RENDERER_GetParam flowchart

#### TDM_RENDERER_TimeStamp

DD_API_TDM_01_013

| Syntax | static void TDM_RENDERER_TimeStamp(XAOMXCodecBase *pBase, OMX_BUFFERHEADERTYPE *pBufHdr) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Calculate timestamp for current input buffer and next buffer. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I/O | Pointer to base codec structure of TDM RENDERER component. |
|  | OMX_BUFFERHEADERTYPE* | pBufHdr | O | Input buffer. |
| Return value | None. |  |  |  |
| Description | - TDM_RENDERER_TimeStamp processing: - Add current timestamp to the Input buffer. - Calculate timestamp for the next buffer. |  |  |  |

[Covers: FD_API_TDM_003]

Figure 3-12 TDM_RENDERER_TimeStamp flowchart

#### TDM_RENDERER_GetParameter

DD_API_TDM_01_014

| Syntax | static OMX_ERRORTYPE TDM_RENDERER_GetParameter(XAOMXCodecBase *pBase, OMX_INDEXTYPE nIndex, OMX_PTR pParam) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Get parameter and save to pParam. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I/O | Pointer to base codec structure of TDM Renderer component. |
|  | OMX_INDEXTYPE | nIndex | I | Index of parameter structure. Valid values: OMX_IndexParamAudioPcm XAOMX_IndexParamAudioTDMRenderer OMX_IndexParamCompBufferSupplier |
|  | OMX_PTR | pParam | O | Pointer to parameter structure want to save. |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorUnsupportedIndex |  | Wrong Index of structure. |  |
| Description | - TDM_RENDERER_GetParameter pprocessing: - Get parameters of OMX_AUDIO_PARAM_PCMMODETYPE, XAOMX_AUDIO_PARAM_TDM_RENDERER, OMX_PARAM_BUFFERSUPPLIERTYPE structure base on nIndex. |  |  |  |

[Covers: FD_API_TDM_004]

Figure 3-13 TDM_RENDERER_GetParameter flowchart

#### TDM_RENDERER_SetParameter

DD_API_TDM_01_015

| Syntax | static OMX_ERRORTYPE TDM_RENDERER_SetParameter(XAOMXCodecBase *pBase, OMX_INDEXTYPE nIndex, OMX_PTR pParam) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Set parameter from pParam and save to parameter structure. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | XAOMXCodecBase* | pBase | I/O | Pointer to base codec structure of TDM Renderer component. |
|  | OMX_INDEXTYPE | nIndex | I | Index of parameter structure. Valid values: OMX_IndexParamAudioPcm XAOMX_IndexParamAudioTDMRenderer OMX_IndexParamCompBufferSupplier |
|  | OMX_PTR | pParam | I | Pointer to parameter structure want to set. |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorUnsupportedIndex |  | Wrong Index of structure. |  |
|  | OMX_ErrorBadPortIndex |  | Port index is invalid. |  |
| Description | - TDM_RENDERER_SetParameter processing: - Set parameters of OMX_AUDIO_PARAM_PCMMODETYPE, XAOMX_AUDIO_PARAM_TDM_RENDERER, OMX_PARAM_BUFFERSUPPLIERTYPE structure base on nIndex. |  |  |  |

[Covers: FD_API_TDM_005]

Figure 3-14 TDM_RENDERER_SetParameter flowchart

#### TDM_RENDERER_ComponentInit

DD_API_TDM_01_016

| Syntax | static OMX_ERRORTYPE TDM_RENDERER_ComponentInit(xf_proxy_t *proxy, OMX_HANDLETYPE hComponent, OMX_PTR pAppData, OMX_CALLBACKTYPE *pCallbacks) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Initialize base codec interface, parameters, PCM and TDM Renderer format. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | xf_proxy_t * | proxy | O | OpenMAX interface proxy. |
|  | OMX_HANDLETYPE | hComponent | O | Pointer to TDM Renderer component handle. |
|  | OMX_PTR | pAppData | I | Pointer to pAppData of TDM Renderer component. |
|  | OMX_CALLBACKTYPE | pCallbacks | I | Pointer to Callbacks of TDM Renderer component. |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorInsufficientResources |  | Initialize base codec interface is fail. |  |
| Description | - TDM_RENDERER_ComponentInit processing: - Initialize base codec interface, parameters, PCM and TDM Renderer format. |  |  |  |

[Covers: FD_API_TDM_001]

Figure 3-15 TDM_RENDERER_ComponentInit flowchart

#### TDM_RENDERER_ComponentDeInit

DD_API_TDM_01_017

| Syntax | static OMX_ERRORTYPE TDM_RENDERER_ComponentDeInit(OMX_HANDLETYPE hComponent) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Destroy TDM Renderer component and private data. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | OMX_HANDLETYPE | hComponent | O | Pointer to TDM Renderer component handle. |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorBadParameter |  | Private data is NULL. |  |
| Description | - TDM_RENDERER_ComponentDeInit processing: - Destroy TDM Renderer component and private data. |  |  |  |

[Covers: FD_API_TDM_002]

Figure 3-16 TDM_RENDERER_ComponentDeInit flowchart

#### TDM_RENDERER_ComponentCreate

DD_API_TDM_01_018

| Syntax | static OMX_ERRORTYPE TDM_RENDERER_ComponentCreate(OMX_HANDLETYPE hComponent) |  |  |  |
| --- | --- | --- | --- | --- |
| Function | Create base component and initialize the TDM Renderer component. |  |  |  |
| Arguments | Type | Name | I/O | Description |
|  | xf_proxy_t* | proxy | O | OpenMAX interface proxy. |
|  | OMX_HANDLETYPE* | hComponent | O | Pointer to TDM Renderer component handle. |
|  | OMX_PTR | pAppData | I | Pointer to pAppData of TDM Renderer component. |
|  | OMX_CALLBACKTYPE * | pCallbacks | I | Pointer to Callbacks of TDM Renderer component. |
| Return value | OMX_ErrorNone |  | Normal return. |  |
|  | OMX_ErrorInsufficientResources |  | Cannot create the base component. Cannot allocate private memory. Initialize the TDM Renderer component is fail. |  |
| Description | - TDM_RENDERER_ComponentCreate processing: - Create the base component. - Allocate the private memory. - Initialize the TDM Renderer component. |  |  |  |

[Covers: FD_API_TDM_001]

Figure 3-17 TDM_RENDERER_ComponentCreate flowchart

# Revision history

| Version | Date | Page | Content | Approved | Changed |
| --- | --- | --- | --- | --- | --- |
| 1.0.0 | Nov 14 2018 | - | First Edition issued | Vu Phan | Ngu Pham |
| 1.1.0 | Dec 10 2018 | - | Add traceability ID | Vu Phan | Ngu Pham |
| 1.2.0 | Jan 03, 2019 | - | Add range for input parameters | Vu Phan | Tien Tran |